### Be 210

# Final Lab Project

## Determining the Heat of Combustion of Oils through Bomb Calorimetry

### April 30, 1998

### Group W5

#### Marc Dworkin

Cindy Hong

Kay Hsu

### David Thakker

##### ABSTRACT

###### Using oxygen bomb calorimetry, the purpose of this experiment was to determine the heat of combustion of vegetable oils.  The heats of combustion were hypothesized to vary among oil types because of compositional differences.  Four benzoic acid calibration trials were performed to calculate the average energy equivalent value, W, for the experimental calorimeter.  The experimental W of 2482  14 cal/C differed from the calorimeter’s literature value for W of 2426 cal/C by 2.21%.  The 95% confidence interval around the mean for the experimental W was 2459  2482  2504 (cal/C), showing that the literature W is statistically different from the experimental W.

##### Using the experimental W, the average heats of combustion for extra virgin olive, peanut, and safflower oil were, respectively: 9434  38 cal/g, 9437  27 cal/g, and 9452  62 cal/g.  The difference between these values was determined to be statistically insignificant through standard differential analysis.  Thus, the experimental data can be averaged to 9440  39 cal/g, indicating that the heats of combustion of oils are independent of fat composition.  The literature value for the heat of combustion of oil by bomb calorimetry was found to be 9450 cal/g, and this differed from the experimental average by 0.11%.  This indicates that bomb calorimetry is an accurate method for determining the gross heat of combustion of oils.

##### TABLE OF CONTENTS

Abstract	1

Table of Contents	2

Background	3

Materials and Apparatus	6

Procedure	7

Results	9

Discussion	13

References	19

Appendix	20

BACKGROUND

Introduction to Bomb Calorimetry

The primary objective of this lab was to determine the heats of combustion of safflower oil, olive oil, and peanut oil using a non-adiabatic Parr Bomb Calorimeter.  The following reaction occurs during the combustion of the oils:

Oil (l) + O2 (g)               CO2 (g) + H2O (l) + energy

In an ideal scenario, all the energy released by the combustion reaction is transferred as heat into the surrounding water.  Thus, the heat of combustion can be obtained from the energy balance:

Qreaction= msample * Hcomb,sample  = mwater * Cp,water * twater

Experimentally, however, not all of the energy released goes to heating the water.  Energy goes into other processes such as heating the metal of the bomb; creating non-standard reaction products, such as nitric acid; and burning the fuse wire used to start the reaction.  Correction factors for these processes must be summed together and subtracted from the calculated energy equivalent, W (cal/C), of the calorimeter.  The energy equivalent is commonly determined through calibration using a sample of benzoic acid.  Benzoic acid is used as a reference material for bomb calorimetry because it burns completely in oxygen, is not hygroscopic, and is readily available in very pure form.  The following reaction occurs during combustion of benzoic acid:

C7H6O2 (s) + O2 (g)                    H2O (l) + CO2 (g) + energy

W=H∗m+e1+e3tThe value of W can then be calculated from the following equation:

Equation 1

where H is the standard heat of combustion of the benzoic acid sample (cal/g), m is the mass of the benzoic acid sample (g), t is the net corrected temperature rise (C), e1 is the correction for heat of formation of nitric acid (cal), and e3 is the correction for heat of combustion of fuse wire (cal).  Knowing the value of W, the heat of combustion for oils can be calculated through the following equation:

H=t∗W−e1−e3mEquation 2

where m is the mass of the oil sample (g), t is the net corrected temperature rise (C), e1 is the correction for heat of formation of nitric acid (cal), and and e3 is the correction for heat of combustion of fuse wire (cal).  [6]

# General Background on Fats

Fats are large molecules constructed from two kinds of smaller molecules: glycerol and fatty acids.  Glycerol is an alcohol with three carbons, each bearing a hydroxyl group.  Fatty acids are long hydrocarbon chains, usually 16-18 carbon atoms in length with a carboxyl group attached to one end.  The nonpolar hydrocarbon bonds make the fats insoluble in water. [1]

A triglycerol, commonly known as fat, is formed when ester bonds link one glycerol molecule to three fatty acids.  The two different types of fats are termed saturated and unsaturated, depending on the structure of the hydrocarbon tails.  Saturated fatty acids have no double bonds between carbon atoms in the hydrocarbon tail and thus have the maximum number of bonded hydrogen atoms.  Most animal fats, such as butter, lard, and bacon grease, are saturated, forming solids at room temperature.  An unsaturated fatty acid has one or more double bonds.  This causes structural kinks at each double bond in the carbon skeleton and a reduction in the number of hydrogen bonds.  Most vegetable fats are unsaturated and are termed oils because they remain liquid at room temperature, since the kinks in the fatty acids prevent the fats from packing closely. Also, this inefficacy in packing allows unsaturated fats to be more readily used in the body than saturated fats, which are easily packed to form adipose tissue.  Unsaturated fats are further categorized as either monounsaturated or polyunsaturated.  Polyunsaturated fats have two or more of their carbon links free (not connected to hydrogen atoms).  Examples are corn oil and soybean oil.  Monounsaturated fats have only one of their carbon links free.  Olive oil is composed of more monounsaturated fat than any other oil (Table 1). [1]

TABLE 1: Fat Composition of Various Vegetable Oils (g fat per 100 g of oil)

| Type of Oil | Polyunsaturated | Monounsaturated | Saturated |
| --- | --- | --- | --- |
| Coconut | 1.69 | 6.59 | 85.17 |
| Cottonseed | 48.14 | 21.3 | 25.59 |
| Corn | 49.28 | 29.32 | 16.43 |
| Olive | 11.18 | 69.72 | 14.04 |
| Palm | 8.31 | 41.64 | 45.27 |
| Peanut | 28.46 | 47.85 | 18.83 |
| Safflower Seed | 72.11 | 12.62 | 10.22 |
| Soybean | 56.73 | 24.26 | 14.05 |
| Sunflower Seed | 49.95 | 31.81 | 13.09 |

# Source: First Supplement to McCance and Widdowson’s The Composition of Foods, p. 72. [9]

# Good Fats versus Bad Fats

In recent years, people have become more calorie conscious for both health and beauty reasons.  One area often targeted is daily fat intake, especially through oils.  Research show that only 30% of one’s daily calorie intake should be fat; however, studies show that in today’s society, approximately 37% of most peoples’ diet is fat [3].  In this “calorie-counting craze,” there is often a misconception about why fat and oils are unhealthy.  Most people scrutinize their calorie intake, often overlooking the composition of the consumed fat.  In actuality, differences in fat composition determine the health effects of oils.

Fat intake correlates to cholesterol levels in the body, which directly affects the risk of heart disease.  Monounsaturated fats, found mostly in olive oils, and polyunsaturated fats, found predominantly in corn oils, have been found to actually lower the cholesterol level in the body.  Therefore, these fats are commonly known as “good” fats.  On the other hand, saturated fats and trans-unsaturated fats, which undergo hydrogenation, are “bad” fats since they significantly increase the cholesterol level in the body.  Common sources of saturated and trans-unsaturated fats are, respectively, animal products or nuts and baked goods or margarine [8]. The “good” and “bad” fats are classified by the chemical structure, which directly influences how they are packed and stored in the body as described in the previous section.

The exact fatty acid composition of fats is another important determinant for the health effects of oils.  Table 2 shows the fatty acid chains most commonly found in oils.  Some of these fatty acids, known as essential fatty acids, cannot be made by the body and must be ingested.  For example, linoleic acid is prime constituent of brain tissue and must be ingested by children in order to ensure normal development [2].  Therefore, consumers should not focus on calorie intake, but composition of fat consumption; some fats actually lower cholesterol levels and other fatty acids are essential for good health.

Table 2:  Most Commonly Found Fatty Acid Chains in Fats

| Type of Fat | Fatty Acid Chain |
| --- | --- |
| Saturated | Myristic Acid |
|  | Palmitic Acid |
|  | Stearic Acid |
| Monounsaturated | Palmitoleic Acid |
|  | Oleic Acid |
| Polyunsaturated | Linoleic Acid |
|  | Linolenic Acid |
|  | Arachidonic Acid |

# Source: First Supplement to McCance and Widdowson’s The Composition of Foods, p. 51. [9]

MATERIALS AND APPARATUS

- Parr Instrument Model 1341 Oxygen Bomb Calorimeter, including stirrer, precision thermometer, and associated components, W= 2426 cal/oC

- Parr Instrument Model 1108 Oxygen Combustion Bomb

- High Pressure oxygen cylinder, equipped with Model 1825 filling connection for bomb

- Parr Model 2901 Ignition Unit

- Parr Pellet Press

- Associated components for test, including sample cups, ignition wire, stands for bomb head and calorimeter cover, thermometer magnifier.

- Ruler

- Top-loading 5 kg capacity balance with a resolution of 0.1 g.

- Desiccator with drierite for drying benzoic acid

- Water (2000 mL) per trial

- Bertolli Extra Virgin Olive Oil, Loriva Peanut Oil, and The International Collection Safflower Oil

- 0.0709N sodium carbonate solution and methyl red indicator for titration

- Burette and Ring stand

- Magnetic Stirrer

- Acetone

PROCEDURE

The bomb calorimeter was assembled using the procedure outlined in the Model 1341 operating instructions.  Initially, the bomb calorimeter was calibrated to verify the energy equivalent, W, of the calorimeter.  A one-gram pellet of benzoic acid was prepared using the Parr pellet press.  The pellet was then carefully placed in the center of the crucible to ensure that it did not touch the crucible’s sides.  Fuse wire, cut to approximately 10 cm in length, was threaded through the electrodes and configured to a point directly above the benzoic acid pellet in the bomb head.  One milliliter of deionized (DI) water was added as a sequestering and absorption agent into the bottom of the bomb.  The bomb head was then lowered into the bomb.  The bomb cap was screwed on, and the bomb was pressurized with pure oxygen up to approximately 30 atm.  Meanwhile, the calorimeter bucket was filled with 2000 mL of a mixture of DI and tap water and lowered into the calorimeter, taking care to ensure that the water in the bucket was approximately 25 oC.  The bomb was then placed in the calorimeter.  The cover with thermometer, which underwent an inversion test, and stirrer were set in place. The temperature of the water was measured every minute for five minutes before ignition.  The temperature was then taken every 45 seconds during the two-minute interval for the period immediately after ignition, and then every minute until it reached a constant temperature for five minutes.

After the experiment, the bomb was slowly depressurized, and the inside was washed with DI water and collected into a beaker.  The washings were titrated with 0.0709 N sodium carbonate and methyl red indicator.  The remnants of the fuse wire were measured to obtain the necessary heat correction factor.  This procedure was repeated three more times with benzoic acid, and the results were plotted as temperature versus time graphs.

# The above procedure was slightly modified when for combustion of oils.  The mass of oil to be used was 0.5 grams, as a result, the length of the fuse wire was increased to approximately 13 cm.  The oils were poured directly into the crucibles, and the crucible was bent to increase the depth and reduce the surface area of the oil (Figure 1).  After the oil samples were combusted, acetone was used to clean the crucibles.

###### Figure 1: Repositioning of the Crucible

# RESULTS

## Standardization of the Bomb Calorimeter

Figure 2: Temperature versus Time during Combustion of Benzoic Acid (Trial 2)

The above graph shows the temperature versus time plot for trial two of the benzoic acid calibration.  The graph contains temperature error bars on the magnitude of 0.015 oC determined from Parr Instruments temperature correction table [6].  The error bars are barely distinguishable on the above graph due to their minute nature.  The equations of the horizontal linear portions before and after the rapid rise period are given.  As expected, the slopes indicate no statistical difference from zero.  The 60% marker indicates the time at which the temperature reached 60% of the total temperature rise.

# Table 3: Measurements and Correction Factors for the Benzoic Acid Calibration Trials

| Item | Trial 1 | Trial 2 | Trial 3 | Trial 4 |
| --- | --- | --- | --- | --- |
| Sample Mass (g) | 1.07 | 0.98 | 1.01 | 1.06 |
| Nitric Acid correction (cal) | 9.60 | 8.45 | 9.60 | 9.28 |
| Firing Wire correction (cal) | 16.01 | 16.81 | 22.36 | 23.00 |
| Corrected Temp (oC) | 2.74 | 2.47 | 2.59 | 2.72 |

Table 3 shows the measurements and correction factors for each of the four benzoic acid combustions.  The above data was used to calculate to experimental energy equivalents, W, using Equation 1 and the heat of combustion of benzoic acid as 6318 cal/g, as given in  Parr Instruments operating instructions manual [6].

# Table 4:  Experimental Energy Equivalent for Benzoic Acid

|  | W (calories/oC) | Systematic error (+/-) |
| --- | --- | --- |
| Trial 1 | 2476 | 12.21 |
| Trial 2 | 2503 | 17.62 |
| Trial 3 | 2473 | 14.38 |
| Trial 4 | 2476 | 14.00 |

The calculated experimental energy equivalents are illustrated in Table 4, derived from the data in Table 3 in conjunction with Equation 1.  The systematic error was computed based on the limitations of the measuring instruments and apparatus.

Table 5:  Statistical Analysis of the Experimental Energy Equivalent of Benzoic Acid

| Average Experimental W(cal/oC) | 2482 |
| --- | --- |
| Literature Value (cal/oC) | 2426 |
| Percent deviation between Trials | 0.57% |
| Percent deviation from Literature | 2.20% |
| Upper 95% Confidence Interval (cal/ oC) | 2458 |
| Lower 95% Confidence Interval (cal/ oC) | 2504 |

Table 5 provides a statistical analysis of the collected data from the four benzoic acid calibration trials.  The literature value, 2426 cal/g, was obtained from the Parr Instruments operating instructions manual [6].  The table shows that the literature value of W does not fall within the 95% confidence interval of the average experimental W and is thus statistically different.

Figure 3: Experimental Energy Equivalents

Figure 3 shows the computed energy equivalents along with systematic error bars obtained from standard differential analysis.  The bold line is the literature value.  The graph illustrates that the uncertainty ranges of all four trials overlap while the literature value falls outside the ranges.  This indicates that the literature value is statistically different from the experimental values.

Combustion of Oils

Figure 4: Temperature versus Time for Safflower Oil Combustion

The above graph shows the temperature versus time plot for the combustion of safflower oil (trial 1).  The graph contains temperature error bars on the magnitude of 0.01 oC, determined from Parr Instruments temperature correction table [6]. The error bars are barely distinguishable on the above graph due to their minute nature.  The equations of the horizontal linear portions before and after the rapid rise period are given.  The slopes indicate no statistical difference from zero.  All of the other oil combustion trials yielded nearly identical graphs.  The 60% marker indicates the time at which the temperature reached 60% of the total temperature rise.

# Table 6: The Average Heats of Combustion for the Tested Oils

| Sample | Average H (cal/g) | Standard Deviation (cal/g) |
| --- | --- | --- |
| Safflower Oil (3 trials) | 9452 | 62 |
| Olive Oil (3 trials) | 9434 | 38 |
| Peanut Oil (3 trials) | 9437 | 27 |
| Overall (9 trials) | 9440 | 39 |

Table 6 lists the average experimental heats of combustion values for the three different types of oils along with the respective standard deviations.  The overall average heat of combustion is also listed for all nine trials.  The raw data used to determine these values for each oil type are found in Appendix A-1 through  A-3.

# Table 7:  Statistical Analysis of the Experimental Heat of Combustion for Tested Oils

| Average Experimental Hc (cal/g) | 9440 |
| --- | --- |
| Literature Value Hc (cal/g) | 9450 |
| Percent deviation between Trials | 0.42% |
| Percent deviation from Literature | 0.11% |
| Upper 95% Confidence Interval (cal/g) | 9471 |
| Lower 95% Confidence Interval (cal/g) | 9410 |
| T-statistical | 2.31 |
| T-critical | 0.69 |

Table 7 provides statistical analysis of the collected data from the combustion of oils.  The literature Hc value for the combustion of oil through bomb calorimetry was obtained from the Pennsylvania State University Department of Nutrition [7].  The table shows that the literature value falls within the 95% confidence interval of the average experimental Hc.  Also, the t-critical value is less than the t-statistical value, which indicates that the literature Hc is not statistically different from the mean experimental Hc.

Figure 5:  Heats of Combustion of Oils

Figure 5 shows the experimental values of heat of combustion for each oil sample along the corresponding systematic error bar.  The error bars account for both the limitations of the measuring instruments and the manufacturing limitations of the calorimeter.  The bold line is the literature value of Hc.  The graph shows that each of the nine trials fall within uncertainty interval of the others.  Also, the literature value is within the range of experimental systematic error since the bold line falls within all the error bars.

DISCUSSION

Since the literature value for the energy equivalent of the calorimeter, W, does not fall within the 95% interval of confidence for the experimental W, the results from the standardization of W demonstrate that the two values are statistically significant.  The literature value, 2426 cal/oC, represents an average W for all of Parr’s Model 1341 Oxygen Bomb Calorimeters, while the experimental value for W is calculated for the specific apparatus used throughout experimentation.  Thus, the differences between the two values are attributed to the age of the experimental calorimeter and the slight workmanship variations in each individual bomb calorimeter unit.

Based on the original hypothesis that chemical composition affects the heats of combustion of oils, the three experimental vegetable oils—extra virgin olive, peanut, and safflower— were chosen based on differences in percent saturated and unsaturated fat.  From Table 1 in the background, olive oil is highest in monounsaturated fat, safflower oil is highest in monounsaturated fat, and peanut oil is highest in saturated fat among the oils available at local grocery stores.

Using the experimental W, the average heats of combustion for extra virgin olive, peanut, and safflower oil were, respectively: 9434  38 cal/g, 9437  27 cal/g, and 9452  62 cal/g.  The 95% confidence level for the olive, peanut, and safflower oils were: 95, 67, and 153 cal/g.  Since the literature value of 9450 cal/g for the heat of combustion of oils by bomb calorimetry fell within the 95% interval of confidences for all three oils, the differences between the average heats of combustion for the three oils were determined to be statistically insignificant.  Thus, the three average heats of combustion were averaged to calculate the experimental value for the heat of combustion of oils as 9440  39 cal/g, which deviated from the literature value by 0.11%.  Therefore, the compositional differences for the three oils had no affect on the average heats of combustion, and the original hypothesis was disproved.

# Justification for Procedural Modifications

Since the tested samples were liquid oils, various procedural adjustments were made as outlined in the Procedure section.  Due to the volatility of oil, the most significant adjustment was in sample size.  Since oil emits a large amount of heat when combusted, the Parr Bomb Company recommended a reduced sample size of 0.7 grams for the first trial.  However, the emitted heat from this sample (6637 cal) was substantially greater than the heat generated from combusting one gram of benzoic acid   (approximately 2480 cal).  To ensure that heat was not lost due to the high magnitude of heat emitted, the sample size of oil was further reduced to release a value of heat more comparable to the heat emitted from the benzoic acid calibration.

The next two samples were 0.3 grams, a mass reduction of approximately 60%.  Only the second sample ignited and emitted 2814 cal, which was well within the range of the benzoic acid.  The reduced sample size required a longer fuse wire, thereby placing it closer to the bottom of the crucible and increasing the chances of the fuse wire short-circuiting.  This suggests that the first 0.3 gram sample failed to ignite because of the fuse wire short-circuited.  The Parr Bomb Company suggested using a wick to ensure the ignition of the small oil samples while preventing the fuse wire from short-circuiting.  Using a wick, however, requires the determination of an additional caloric correction factor, which must be attained through experimentation.  Therefore, instead of using a wick, the crucible was tilted forward to concentrate the oil to one side of the crucible, while increasing its depth and reducing its surface area (as illustrated in Figure 1).  This minimized short-circuit risks by increasing the distance between the fuse wire and the crucible, and it also brought the oil into closer vicinity of the fuse without direct contact.

The calculated values for the heats of combustion of the oils determined that the size of the sample does not significantly affect the heat of combustion of the oil since the calculated value for the 0.3 gram sample was only 1.1% lower than the 0.7 gram sample.  This proved that the 0.7 gram sample was an accurate trial despite the magnitude of the energy release and the range of the calibration standard of the bomb calorimeter.  Therefore, the sample size was increased to 0.5 gram for remaining trials to make the test samples more manageable.

Sources of Error and Error Analysis

In this experiment, the only real source of error was the human error in taking the various measurements, such as length, mass, time, and temperature.  However, to minimize the effect of the error on the results, specific tasks were divided among the lab members to ensure that every measurement was read with the same precision.  This specialization of tasks also increased the efficiency of the experimentation.  Another possible source of error was the environmental conditions of the laboratory, however the non-hydroscopic nature of oils eliminates any influence that may result from a moist environment.

The systematic experimental errors due to human and measurement equipment limitations for the mass of the sample, the temperature rise, nitric acid correction, and fuse wire correction were determined using standard differential analysis. The error resulting from the measurement of the mass of water in the bomb was determined to be one cal/oC based on the accuracy of the scale used.  This increased the error of W, the effective heat capacity of the calorimeter apparatus by one cal/oC, the heat capacity of one gram of water.  For the oil trials, standard differential analysis was used to determine the effect of the above-described error in W on the experimentally acquired heats of combustion.

The systematic uncertainty interval for the rate at which temperature rose during the equilibrium periods before firing and after rapid rise was determined by the standard error obtained through a regression analysis on the time vs. temperature line for those periods.  Based on observations made during the first few trials, the experimental procedure was modified by assuming that the temperature rate change during the before firing and rapid rise periods was statistically indiscriminate from zero.   Therefore, an average value of error for the pre-modified procedure trials was used in determining the temperature error.

Validity of Data

The validity of the data obtained from combustion of the oil samples was shown in three ways.  The first method was the 95% confidence interval test.  Table 5 shows that the literature value for Hc for oil falls between the upper and lower 95% confidence interval around the mean for the nine experimental trials.  This demonstrates that the literature value is significantly insignificant from the mean experimental value at the 95% confidence level.  The second proof of validity was the test of significance.  In this test, the statistical t-value (calculated based on 8 degrees of freedom and a 0.05 probability level) was compared to the critical (experimental) t-value (calculated based on the literature and mean values of Hc).  Table 7 shows that the t-critical value is less than the t-statistical value.  This indicates that there is no significant difference between the literature value for Hc and the mean experimental value for Hc.  The third test of validity is shown in Figure 5.  This plot shows that the literature value for Hc falls within the error bars for all nine trials, indicating that it is within the systematic uncertainty interval for the entire experiment.

A 10% Disparity and Limitations of Bomb Calorimetry

Along with initially hypothesizing that the chemical structure of oils affected their heats of combustion, it was expected that the experimental heats of combustion would be equivalent to the calorific value for “Total Fat” given on the bottle labels.  The label value of 120 g/serving size of one tablespoon converts to 8571 cal/g.

With a 10.1% disparity between the experimental heats of combustion and the label values, one of these values was initially concluded to be wrong. After contacting the Food and Drug Administration (FDA), the American Oil Society, and other sources, it was discovered that neither value was wrong, but extreme caution had to be used when comparing the experimental and label values for the following three reasons:

First, the two values were derived from two drastically different methods.  The method discussed above was used to obtain the experimental gross heats of combustion.  Label values, on the other hand, are calculated values, based purely on accepted FDA regulations with no experimental backing.  Crisco, Bertolli and other oil companies never use bomb calorimetry to quantify the energy released by their oils.  Instead, the food industry and the FDA accept certain chemical methods to determine the grams of protein, fat, and carbohydrates in food samples, and these amounts are multiplied by factors assumed to represent the number of calories produced in the body by one gram of protein, fat, and carbohydrate.  The fat factor is calculated by averaging the fat value and calories over a spectrum of foods, and the FDA accepts that there are 9.1 calories per gram of fat. [9]

Second, although the label value is supposed to reflect what happens in the body, the bomb calorimeter does not model the human body well.  Oils in the bomb were combusted at 30 atm, and the combustion was nearly instantaneous.  Clearly, conditions in the body are quite different.

Thirdly, the bomb measures the heat of combustion for the complete breakdown of the oil sample.  In the body, however, not all of the oil consumed generates calories.  Some fat is stored in adipose tissue, some fat is needed for oil glands and membrane lipid protein, and some fat never gets absorbed into the system and is excreted.  Since the label calorie value only reflects the portion of oil that is metabolized, it is inherently different from a complete heat of combustion value obtained through calorimetry. [1]

With these three reasons in mind, the following justification for the 10.1% difference between the literature and experimental values for the heat of combustion of oils is proposed.  Published literature states that 94 -98% of ingested fat is absorbed into the intestinal tract [10].  Assuming that all the absorbed fat is used for metabolism, storage, and other biological uses, this suggests that the remaining 2 to 6% of ingested fat is secreted or excreted and not a calorie component in fat metabolism.  The label value accounts for the calories released through the metabolism of fats, whereas the experimental value assumes 100% efficiency in fat combustion.  Thus, in addition to the energy released through fat metabolism (label value component), the experimental value also includes the potential energy for all the unmetabolized fats.  Therefore, the experimental value should exceed the label value by more than 2 to 6%.

Even with this speculation, the difference between the experimental and label values is not fully accounted.  We attribute the extra energy to the heat of combustion of anti-oxidants and preservatives contained in the oils.  Anti-oxidants are added to oils in trace amounts to prevent oils from oxidizing due to heat, light, the presence of oxygen, and contact with metals.  Common anti-oxidants for oils include ascorbic acid, with the ability to bind to free oxygen in oil, and octyl- and dodecyl esters.  The combustion of all these organic compounds in the bomb calorimeter contributes to a higher gross heat of combustion for oil than label values. Additionally, the labels for most oils claim to have no preservatives, even though FDA regulation allows foods which contains less than several parts per million to be labeled as “no preservatives or additives.”  Common preservatives are chloride derivatives, which also would add additional energy to the experimental gross heat of combustion. [4]

With these considerations in mind, we conclude that bomb calorimetry is an effective method for computing the gross heats of combustion for oils.  However, since the bomb simply measures the heat released in a complete combustion reaction, it cannot give any information about the composition of oils.  Therefore, bomb calorimetry is not a practical technique for biomedical applications since it is not the calorie values of oils that are important for good health, but their fat compositions.  This is affirmed by the common usage of other techniques such as gas chromatography and the Von Liberman technique in determining the calorie content and compositions of fats in food sources today [9].

REFERENCES

[1]  Campell, Neil. Biology.  New York:  The Benjamin/Cummings Publishing Company, Inc., 1993.

[1]  www.canola-council .org/nutrin/dietaryfat.html.

[2]  “Food Insight Reports:  Sorting Out the Facts About Fat.”  Washington D.C.:  International Food Information Council, 1989.

[3]  Goodwin, R.W.L.  Chemical Additives in Food.  Boston: Little, Brown, and Company, 1967.

[5]  www.heartinfo.com/News97/fatnejm112197.htm.

[6]  BE 210 Bioengineering Laboratory II Laboratory Manual, Spring 1998.  “Experiment

6: Equilibrium Electrochemistry.”

[7]  www.nutrition.hhdev.psu.edu

[8]  Passmore, R., and Eastwood, M.A. Human Nutrition and Dietetics.  New York:  Churchill Livingstone, 1986.

[9]  Paul, A.A. and D.A.T. Southgate First Supplement to McCance and Widdowson’s The Composition of Foods. New York: Elsevier/North-Holland Biomedical Press, 1980.

# [10]  Weiss, Theodore J.  McGraw-Hill Encyclopedia of Science and Technology. New York: McGraw-Hill, Inc., 1992.

# APPENDIX A

Table A-1:  Safflower Oil

| Item | Trial 1 | Trial 2 | Trial 3 |
| --- | --- | --- | --- |
| Sample Mass (g) | 0.711 | 0.715 | 0.292 |
| Nitric Acid correction (cal) | 9.92 | 6.60 | 3.25 |
| Firing Wire correction (cal) | 22.36 | 21.62 | 20.10 |
| Corrected Temp (oC) | 2.73 | 2.75 | 1.11 |
| Energy Equivalent of Calorimeter (cal/oC) | 2482 | 2482 | 2482 |

This table shows the measurements and correction factors obtained from the combustion of safflower oil.  The average value of Hc for safflower oil given in Table 4 was calculated using the data in this table in conjunction with Equation 2.  The energy equivalent represents the average experimental value obtained from the benzoic acid calibration trials.

# Table A-2:  Peanut Oil

| Item | Trial 1 | Trial 2 | Trial 3 |
| --- | --- | --- | --- |
| Sample Mass (g) | 0.510 | 0.497 | 0.508 |
| Nitric Acid correction (cal) | 7.3 | 6.6 | 6.9 |
| Firing Wire correction (cal) | 13.73 | 19.50 | 19.92 |
| Corrected Temp (oC) | 1.95 | 1.90 | 1.94 |
| Energy Equivalent of Calorimeter (cal/oC) | 2482 | 2482 | 2482 |

This table shows the measurements and correction factors obtained from the combustion of peanut oil. The average value of Hc for peanut oil given in Table 4 was calculated using the data in this table in conjunction with Equation 2.

#### Table A-3:  Olive Oil

| Item | Trial 1 | Trial 2 | Trial 3 |
| --- | --- | --- | --- |
| Sample Mass (g) | 0.503 | 0.506 | 0.504 |
| Nitric Acid correction (cal) | 6.65 | 6.71 | 6.88 |
| Firing Wire correction (cal) | 20.24 | 20.24 | 21.05 |
| Corrected Temp (oC) | 1.93 | 1.93 | 1.93 |
| Energy Equivalent of Calorimeter (cal/oC) | 2482 | 2482 | 2482 |

This table shows the measurements and correction factors obtained from the combustion of olive oil. The average value of Hc for olive oil given in Table 4 was calculated using the data in this table in conjunction with Equation 2.