Determination of the Iron Content in Prune Juice

Group R3	Final Report

- Grace Doe

- Michael Giron

- Michael Kulon

- Julia Wisniewski

# Table of Contents

Abstract	2

Background	2

Methods	3

Results	6

Discussion	12

Error Analysis	14

Conclusion	14

Appendix 1: Tables	17

Appendix 2: Equations	19

References	20

# Abstract

The iron content of SunSweet® prune juice was determined using atomic absorption spectroscopy techniques.  The juice was digested with 12N HCl, and the direct measurements were corrected for matrix effects by method of additions.  The iron content was determined to be 6.0 ppm with a 0.84% 95% confidence interval.  This finding falls within the range published by SunSweet® manufacturers (7.5 ppm  20%).  Various concentrations of prune juice were used in the experimental procedure.  The detected amount of iron was not skewed by this method.  It was concluded that for the concentrations tested, the induced pH was sufficient to solvate the iron.  The kinetics of digestion was also investigated.  The detected amount of iron increased with digestion time, leveling off after a period of approximately 90 minutes.

# Background

- Literature Values

According to the SunSweet® prune juice information label, the USRDA determined that for each 240g serving size, 1.8 mg of iron was available. Using this value, the iron concentration of prune juice was found to be 7.5ppm with a 20% error. The 20% error is indicative of the variation iron content due to the biological source of prune juice. These variations result from inconsistencies in the natural prune, various prune juice batches, and method of processing.

- Problems Studied in the Experiment

In order to accurately determine the iron content in SunSweet® prune juice, procedural corrections, such as drift, time digestion, and matrix effects were investigated.

Due to the inherent fluctuations in the process of atomic absorption spectroscopy, the readings are expected to drift during the experimental procedure. In order to account for such fluctuations, a procedural correction was devised based on  measurement of a known standard stock iron solution between each experimental measurement.

Time digestion was another issue of concern.  It is conceivable that the added 12M HCl may require some time to completely react with the prune juice. In order to correct for this, several digestions of varying durations were made. It was anticipated that the added HCl, would hydrolyze protein aggregates, and convert iron compounds to a soluble form.  It was necessary to experimentally determine the relationship between detected concentration and acid reaction time period.

Additionally, the viscosity effect was investigated. Due to the difference in viscosity between standard solutions and the various concentrations of prune juice, the differences in aspiration rate may affect the final measured iron absorbance. Thus, to adjust for the rate of up-take into the AAS, a correction factor was necessary. First, the method of additions, commonly known as spiking, was utilized to determine if in fact viscosity was significant. This analytical chemical procedure is useful for checking the accuracy of an element concentration in the presence of foreign substances (9). Dilutions of a known iron concentrated solution are measured for iron absorbance. If viscosity is not a factor, the same concentration for each diluted absorbance should be obtained. However, if found to be significant, a correction factor (equation 4 and 5) is required.

## Theory of Method

There are numerous methods for determination of iron content in food.  These include Electrothermal Atomic Absorption Spectrometry (B2), Radiochemical Neutron Activation Absorption (B1), and Total-reflection X-ray Florescence (B3). However, these methods, due to limited linear ranges, excessively rare and expensive equipment, and focus on solid substances, are not practical.  The following discussion focuses on atomic absorption spectroscopy.

The Perkin-Elmer Model AA 4000 Atomic Absorption Spectrophotometer was designed to measure trace element absorbances in various specimens. According to quantum theory, when an element is excited, it emits discrete wavelengths of light. When the wavelength of the AAS is fixed at 248.3 nm absorbance measurements are indicative of the concentration of the iron in the aspiration sample.  When all parameters are set accordingly, the Beer-Lambert Law can be used to calculate the concentration of iron in the sample from the measured absorbance by equation 1(4).

|  | A = log T = el C b | (Eq. 1) |
| --- | --- | --- |

Methods of sample preparation range from concentrated acid digestion to different ashing techniques followed by decanting and resuspension in acid.  High lipid samples require more acid for destruction whereas high moisture/low lipid samples require less acid.  The food digestion followed by the FDA for analysis by Inductively Coupled Argon Plasma Emission (ICP) and Hydride Generation Atomic Absorption Spectroscopy (HYAAS) techniques consists of a mixture of nitric, perchloric and sulfuric acids.  Historically this digestion scheme is successful for samples of 2 to 5g depending on the matrix (e.g. high versus low lipid).  Initially the easily oxidized materials are destroyed in the nitric acid boiling range.  Sulfuric acid acts as a catalyst for the nitric acid and perchloric acids in digestion (3).

With the imposed safety regulations for the Bioengineering laboratory at the University of Pennsylvania, prune juice was chosen for analysis based on its zero lipid and high iron content. According to Dr. Lyon, Director of Chemistry of the American Food Processing Association, prune juice is a good test specimen. But Lyon also warned that HCl could possibly fail to remove organic compounds from the prune juice (2).  Having this information, it was deemed necessary to test the efficacy of HCl digestion (1).

# Methods

- Apparatus and Reagents

- Perkin-Elmer Model AA 4000 Atomic Absorption Spectrophotometer

- Standard solutions 1000ppm iron nitrate

- Disposable plastic beakers for aspiration of samples

- 100, 500 ml volumetric flask

- Parafilm

- Micropipettes

- Beckman automatic pipetter

- SunSweet® Prune Juice

- Other Assorted Glassware

- Calibration

The iron concentration in the sample will be directly measured using the Perkin-Elmer Atomic Absorption Spectrophotometer.  The machine was calibrated for iron at the start of each day.  A small degree of drift was expected to occur, since it is conceivable that small particles may get trapped in the atomizer, affecting the rate of flow and consequently influencing the final measurements.  Therefore re-calibrations were made in between trials to account for the possibility of drift.

The calibration was made using a 1000ppm-iron nitrate solution according to the dilution scheme detailed in table 7.  5ml of 15M nitric acid were added to the dilutions in order to prevent the formation of insoluble iron compounds (mainly hydroxides).  Those solutions (500mL each) were bottled and used throughout the duration of the experiment.  A solution of 5ppm was made fresh every day and compared with the equivalent concentration of the original standard.  If the readings on the spectrophotometer were significantly different when measuring these two solutions, it would have been an indication that the dilutions have changed in concentration, presumably due to formation of precipitates.  In this case, all dilutions required for the calibration curve would have to be prepared anew and the calibration made using the new solutions.  However, this behavior was not observed.

- Digestion Time

Prior to testing, each juice sample was digested with 5 ml of 12M HCl acid.  Digesting each sample ensures that the iron remains in solution and does not form insoluble compounds.  While insoluble compounds do not decrease the amount of iron in the sample, the iron present may not be sufficiently liberated to be accurately detected by the AA Spectrophotometer.

As mentioned earlier, the Perkins-Elmer procedure for determining the iron content in juice does not explicitly state the time that the juice is to be digested with concentrated HCl acid (6).  To test this phenomenon five digestions of approximately 5-ppm iron were prepared.  After digestion time of 5, 20, 40, 90, 120 min, each digestion was diluted to 100 ml and tested for absorbance.  If there is no time dependency to digestion, each of the subsequent readings should be the same.  In the case that the absorbances are not the same, the digestion time can be increased until the resultant absorbances become constant.  With this information, a proper digestion time was determined to obtain the maximum amount of iron in the juice while minimizing the time wasted in unnecessarily long digestions.

- Viscosity & Available Iron by Digestion

Since various concentrations of juice were used for the digestions, it was conceivable that certain aspects of this procedure might skew absorbance readings.  By concentrating the juice, concentration of proteins, sugars, and other solutes increased.  This tended to increase the viscosity of the final sample that was measured with the spectrophotometer.  Since the rate of flow through a tube is inversely proportional to viscosity of the liquid, the aspirator was not necessarily able to maintain a constant rate of solution uptake.  The same amount of acid (5mL, 12M HCl) was used to digest the different masses of prune juice.  It is conceivable that more buffering substances were  present at higher juice concentrations, thus hindering the formation of soluble iron.

In order to detect if these effects are significant, the different digestions of juice were spiked with 5 different amounts of iron (1 ppm to 4 ppm) (1).  The data was corrected according to the equation 2.

|  |  | (Eq 2) |
| --- | --- | --- |

|  | C0: corrected value for concentration of iron in the digestion. C: change in concentration due to spiking. Ro: absorbance reading of iron in the digestion (unspiked). Rs: absorbance reading of iron in the digestion after spiking. : proportionality constant between absorbance and iron concentration |
| --- | --- |

The reasoning behind equation 2 is that if only a fraction of the spike is detected, the original unspiked reading corresponds to the same fraction of iron that is really present in the sample.  After 5 trials absorbance was plotted as a function of spike concentration. Linear regression analysis provided an equation for the linear relationship.  The y-intercept of the equation was corrected according to equation 3, but the linear regression analysis provided other useful statistics.  This method is equivalent to using equation 3.

|  |  | (Eq 3) |
| --- | --- | --- |

It was assumed that the amount of acid added (5% by volume, 12M HCl) was sufficient to overcome any buffering effects of the juice and induce an adequately low pH so that no insoluble iron hydroxide was present.  This assumption would most likely fail for extremely high juice concentrations. For the concentrations used (5  concentrate in the most severe case), it is expected to hold, but only by using a pH meter could this be proven.

# Results

- Standard Calibration Curve

The standard calibration curve of concentration vs. absorbance for weeks 1-3 is presented in figure 1. Since the relationship between concentration and absorbance (for concentrations between 1.5 and 10ppm) was linear, the Beer-Lambert Law was valid for evaluating data. The slope of the standard curve represents the constant b in the Beer-Lambert Law.  Table 1 summarizes the squared coefficient of correlation (R2) for each line fit to the data.  Upper and lower 95% confidence limits for the slope (constant b) are also shown.

### Figure 1. Three week calibration of standard stock solution

| Week | Slope | R2 value | Upper 95% confidence limit | Lower 95% confidence limit |
| --- | --- | --- | --- | --- |
| 1 | .0395 | .9998 | 0.038794 | 0.040234 |
| 2 | .0235 | .9998 | 0.023092 | 0.02347 |
| 3 | .0343 | .9998 | 0.034855 | 0.033735 |

Table 1: Table summary values obtained for Weeks 1-3 calibration curve of iron stock

- Digestion-Time Test

Figure 2.  Results of digestion effectiveness as a function of time.

A plateau occurs after approximately 1 hour.

| Mass of Prune Juice | Measured Absorbance | Calculated [Fe] of undiluted prune juice |
| --- | --- | --- |
| 70.53 | 0.121 | 4.515 |
| 70.51 | 0.134 | 4.926 |
| 70.80 | 0.12 | 4.932 |
| 70.80 | 0.118 | 4.927 |
| 70.85 | 0.133 | 4.991 |
| 70.41 | 0.128 | 4.947 |

Table 2.  Results of digestion effectiveness as a function of time.

A plateau occurs after approximately 1 hour.

The data obtained during the digestion time test (figure 2, table 2) suggests that the duration of digestion is important.  The findings were used only to determine that 90 minutes was necessary for digestion, and were not corrected for viscosity effects.

- 90-minute HCl Digestions Compared to Undigested Juice

The effect of digesting the prune juice samples with 12M HCl for 90 minutes was also measured as a percent increase in the amount of iron detected with spectroscopy. All mass measurements were carefully recorded and all samples were diluted with deionized water in a final volume of 100ml. To see this data, refer to appendix table 8.

From the data in appendix table 8, the following graph was constructed.  Note that the data was not corrected for viscosity effects.  The only variable that was considered in this section was the effect of 90-minute acid digestion.

Figure 3. The  concentration of detectable iron is plotted against the mass of samples of digested and undigested prune juice

- Corrected 90-minute Digestions

Figures 4a&b.  The concentration of iron versus the concentration of spike added.

Using its associated calibration curve as well as drift correction for each point, figures 4a and 4b show sample iron concentrations plotted against the concentrations of spike added to each digestion of juice for the second and third week, respectively.  With greater volumes of raw juice added, the slopes of the spiking regression diminished from the expected slope of 1.

Figures 5a&b.  The calculated iron concentration plotted versus the mass of juice used, uncorrected for viscosity effects.

Using the zero spike data and the dilution factors, the iron concentration was calculated for undiluted prune juice and then plotted against mass of juice used, again for the second and third week, respectively.  These calculated values again tended to fall as the mass of juice increased.

Figures 6a&b. The calculated iron concentration plotted versus the mass of juice used, corrected for viscosity effects.

Figures 6a and 6b show the calculated iron concentration in undiluted prune juice utilizing the spike data for the second and third week, respectively.  These slopes were not significantly different from zero.

# Discussion

- 90-minute 12N HCl Digestions Compared to Undigested Juice

Since the solubility of iron is ultimately a function of pH, the hypothesis was that the addition of HCl would increase the amount of soluble iron in the sample and make iron more readily detectable at the time of aspiration (9).  5ml of HCl was chosen based on procedures published by the manufacturer of the spectrometer used  for  the experiment. Figure 2 shows that there is a time dependent relationship between the concentration of detectable iron and the time of HCl digestion. At 90 minutes, the graph of iron concentration versus time plateaus indicating that 90 minutes is sufficient for digestion.  Background literature indicates that HCl is one of three acids commonly used to increase the effectiveness of measuring iron with atomic absorption spectroscopy (9). Explanation of this time dependence relationship was discussed previously in the background.

Results of the digestion time test are confirmed by the observations presented in figure 3. To discuss this effect, a line can be fit through the two data sets and a relationship between mass of prune juice and detected iron concentration derived.  The percent increase in detectable iron due to addition of HCl can then be quantitated by comparing the two slopes.  This was done in figure 7.

Figure 7. The concentration of detectable iron is plotted against the mass of samples of digested

and undigested prune juice

For undigested prune juice, the detectable iron concentration (y in ppm) decreases by equation 4.  For digested prune juice, the detectable iron concentration (y in ppm) decreases by equation 5. The variable x is the mass of prune juice that was diluted to 100ml. When HCl was used to digest the high concentration samples for 90 minutes before aspirating, the slope of detectable iron concentration versus mass of sample increases by 64%.  This increase represents a percent reduction in interference associated with higher concentrations of prune juice.

The remainder of the discussion focuses on the interference due to differences in viscosity between the standard and the juice.

- Corrected 90-Minute Digestions

As shown in figures 4a and 4b, the slopes of the iron concentration versus spike graphs tended to diminish as the volume of raw juice used in the digestion increased, with a minimum slope of 0.8989 for the 65.8ml volume of juice.  Without considering viscosity, adding an iron spike should increase the total sample concentration by the same value (a 1 ppm spike should raise the sample concentration by 1 ppm) and the slope of each of these regressions should be equal to unity.  Because this was not the case for the trials that used greater volumes of juice, it was hypothesized that the viscosity of these samples slowed down the rate of intake into the aspirator tube of the AAS.  This in turn resulted in concentration readings that were less than the actual concentrations of the solutions.

This trend was also seen in the plot of the zero spike data that was multiplied by the dilution factor to obtain the concentration of iron in undiluted prune juice (see Figures 5a and 5b).  Again concentrations that should have been constant regardless of the mass of juice used (i.e. a slope of zero) actually decreased with increasing juice volume (slopes were –0.0207 and –0.0215 for the second and third week, respectively).  However equations 2 and 3 were used to normalize the data and the results in appendix table 9, which show the corrected iron concentrations, were used to construct the graphs in figures 6a and 6b.  Because the slopes of these graphs were very close to zero (week 2 data was not statistically different from zero, week 3 data differed from zero by only 0.001616 ppm ml-1, see appendix table 10) the final concentration of iron in undiluted prune juice did not decrease in digestions with greater volumes of juice used. Thus spiking provided an effective and valid procedure for determining the final concentration of iron in viscous organic based solutions.

Using Excel’s descriptive statistics package to analyze the corrected data, the iron concentration in undiluted prune juice was found to be (6.00  0.05) ppm leading to 95% confidence interval of only 0.837%.  This is an exceptional uncertainty considering that the predicted uncertainty from the equipment was 1.51% (see error analysis table 3).  However the predicted uncertainty stemmed mainly from the 1.5% published uncertainty associated with the AAS, and experimentally the full extent of this uncertainty was not seen.  Absorbance readings only varied one or two thousandths of an absorbance unit.  This drastically decreased the uncertainty associated with each measurement and resulted in the low experimental uncertainty that was observed.

# Error Analysis

Practical Error

Several sources contributed to the uncertainty in the results in this experiment.  Preliminary calculations were made in choosing glassware and transfer apparatus that would reduce error to an acceptable amount.  Acceptable, for the USRDA is defined as  20%.  Acceptable, for group R3 was defined as 1.51%. Using the practical error equation this number was deemed reasonable based on the calculations summarized in table 3 (4).

Table 3. Predicted Practical Error of Method

| Error Source | Maximum Precision |
| --- | --- |
| balance | 0.004% |
| volumetric | 0.20% |
| AASpec | 0.82% |
| METHOD | 0.84% |

Table 4.  Determined Practical Error of Method

Considering that the combined precision of the balance and 100ml volumetric had to contribute 0.01% uncertainty in recorded values, it was possible to work backwards from table 3 and determine the actual precision of the AAS to be 0.82%.  Calculations are presented in table 4.

# Conclusion

Several conclusions were derived from the findings presented in this report and from preliminary research that was completed prior to the experiment.  Based on the precision associated with the results of iron determination prune juice, the proposed method was proven successful and effective in completely its main objective.  This suggests that the same procedure might be used for testing other organic aqueous solutions with approximate iron concentrations in the range of 7.5ppm in order to obtain a higher precision measurement.

- Method

Given the narrow confidence limits and the consistency of the acquired data, the experimental method was successful.  It is reasonable to conclude that it has a high probability of success for detection of other elements in juices.

- Economic Analysis

Given proper R&D and Perkin-Elmer Model AA 4000 AAS, a .84% precision of Iron absorption can be attained assuming there is a linear relationship between cost and precision. The 0.84% precision measurement coincides with the determined $7,140 total expense as indicated in table 5. Utilizing this coordinate point and the initial point (0 hrs, 20%), figures 8 and 9 were obtained. At the same previous retail price of 3.25 dollars, only addition 2,200-bottle sales are required for a net profit of 0. For more realistic predictions, a market survey can be conducted to determine the possible rate of return at variable prices.

Overall, in the case of SunSweet® prune juice, the additional .84% precision is neither necessary nor likely to increase profit. Upon utilizing a t-paired test, it was discovered that there was a difference in the final measurement of iron content. However, in the prune juice  significant in terms of daily value use. Thus, .84% precision determination is unnecessary in this case. However, were the issue of precision applied to the determination of trace elements in medical devices / procedures, precision would be essential. The precision of the trace element in biomedical surgery (i.e. joint replacement) and similar situations has a direct impact on the patients’ life in addition to profitability. Thus, the necessity of the .84% precision requires a close examination of the situation and possible profitability.

| Time Break Down | 1 Week Period | 3 Week Period | Total Time (hours) |
| --- | --- | --- | --- |
| Planning (hours) | 6 | 18 | 72 |
| Research (hours) | 3 | 9 | 36 |
| Presentations (hours) | 5 | 15 | 60 |
| Experimentation (hrs) | 4.5 | 13.5 | 54 |
| Data Analysis (hours) | ----- | ----- | 16 |
| Total Cost (dollars) | 555 | 1665 | 7140 |

Table 5. Total Time and Cost break down of .84% precision

| Precision (%) | Excess Labor (hours) | Excess Cost ($) | Excess Quantity Sale |
| --- | --- | --- | --- |
| 20 | 0 | 0 | 0 |
| 10 | 3731.343 | 3731.343 | 1148.106 |
| 5 | 5597.015 | 5597.015 | 1722.158 |
| 2.5 | 6529.851 | 6529.851 | 2009.185 |
| 1.25 | 6996.269 | 6996.269 | 2152.698 |
| 0.84 | 7149.254 | 7149.254 | 2199.77 |

Table 6.  Labored and cost required per doubling of precision

Figure 8. Precision – Cost relationship

Figure 9. Precision - Time relationship

- Future Implications

Background research showed the biomedical significance of the determination of iron content in food.  Various sources also implicate the relevance of an investigation of the increased iron concentration in food as a result of iron cookware.  The occurrence of oral iron overload, a condition which was discussed in the background, is very rare except in southern Africa where alcoholic beverages are prepared in iron drums and pots (11).  The mean iron content in such brews has been found to vary between 40-80 mg/L and the pH of the beverage is very low.  Most iron is consumed in the beer or in maize porridge cooked in iron pots.  What is interesting is that iron overload is only a problem for beer drinkers.  Presumably, the formation of soluble iron complexes during fermentation leads to better absorption of iron in the body.  This is similar to the effect of adding HCl to the raw prune juice samples to increase the amount soluble iron.

The implication for future endeavors would be to follow the protocol already outlined, but rather than add 5ml of HCl all at once, add 1ml of HCl at a time and monitor pH changes in the solution.  Then, the amount of soluble iron can be plotted against the pH.  A functional relationship between soluble iron and pH would be allowed for more effective preparation of acid digestions.  That is, the timely task of boiling down juice samples to 20ml could be eliminated and instead of maintaining constant volume of the samples, maintaining constant pH among samples would provide a more time efficient and precise control method.

Numerous acids are suggested by the FDA and USRDA procedures for sample preparation for AAS.  The results of this experiment showed that acid was necessary, but because there is not a fume hood available in the lab, HCl was the only acid that could be used safely by students.  A separate investigation (with the availability of a fume hood) might reveal whether the type or combination of acids used in setting up a digestion has an effect on results.

# Appendix 1: Tables

| Stock (ppm) | Dilution Factor | Volume of Stock (ml) | Final Volume (ml) | Expected Absorbance by Eqn. * | Final Standard (ppm) | Measured Absorbance Week 1 | Measured Absorbance Week 2 | Measured Absorbance Week 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1000 | 66.7 | 7.50 | 500 | 0.540 | 15.0 | 0.366 |  |  |
| 1000 | 100.0 | 5.00 | 500 | 0.360 | 10.0 |  |  | 0.350 |
| 1000 | 166.7 | 3.00 | 500 | 0.216 | 6.0 | 0.247 | 0.147 | 0.216 |
| 1000 | 181.8 | 2.75 | 500 | 0.198 | 5.5 |  |  |  |
| 1000 | 200.0 | 2.50 | 500 | 0.180 | 5.0 | 0.206 | 0.122 | 0.180 |
| 1000 | 222.2 | 2.25 | 500 | 0.162 | 4.5 |  |  |  |
| 1000 | 250.0 | 2.00 | 500 | 0.144 | 4.0 | 0.168 | 0.100 | 0.147 |
| 1000 | 285.7 | 1.75 | 500 | 0.126 | 3.5 |  |  |  |
| 1000 | 333.3 | 1.50 | 500 | 0.108 | 3.0 | 0.128 | 0.076 | 0.110 |
| 1000 | 400.0 | 1.25 | 500 | 0.090 | 2.5 |  |  |  |
| 1000 | 500.0 | 1.00 | 500 | 0.072 | 2.0 | 0.090 | 0.053 | 0.078 |
| 1000 | 666.7 | 0.75 | 500 | 0.054 | 1.5 |  |  |  |
| 1000 | 1000.0 | 0.50 | 500 | 0.036 | 1.0 | 0.048 | 0.029 | 0.041 |

- Table 7: Dilution Scheme for Standard Curve

Table 8: HCl versus No HCl Comparative Data

Table 9: Viscosity Corrected Data

Table 10: Slope analysis

# Appendix 2: Equations

|  | y = -0.0427x + 7.4897 | (Eq. 4) |
| --- | --- | --- |
|  | y = -0.0152x + 6.6773 | (Eq. 5) |

# References

1. Al Giandomenico,  Lab Coordinator.  Personal Interviews and Electronic Mail Communications. March 1999.

2. Lyon, Roy Ph.D. Director of Chemistry, National Food Processing Association.  Telephone Interview. 17 March 1999.

3. Fitzpatrick, W.H. Ph.D. Nutrition Research in the USSR 1961-1970,  Washington D.C., Geographic Health Studies Program.  John E. Fogarty International Center for Advanced Study in the Health Sciences. 1972, 69-70 and 110.

4. Litt, Dr. Mitchell; and Giandomenico, Albert.  BE 210 Bioengineering Laboratory II, Laboratory Manual.  University of Pennsylvania, Spring 1998.

5. Food and Drug Administration’s Total Diet Study, Ternary Acid Digestion. (Kansas City District Laboratory. Lenexa, KS,  November 30, 1993)

6. Perkin Elmer Corporation, Manual for Model 4000 AAS, pages FP-1, FP-2, FP-7

7. Official Methods of Analysis of the A.O.A.C.

8.  “Qualitative Spectrographic Analysis in the Arc, with Graphite Electrodes”.  W.C. Pierce, O. Ramirez Torres, and W.W. Marshall, University of Chicago, Chicago, Ill.  Industrial and Engineering Chemistry, Vol. 15  January 15, 1940, 41-45.

9. Reynolds, R.S. Atomic Absorption Spectroscopy: A Practical Guide,  London,  Griffin. 1970.

10. Borch-Ionsen, Berit.  “Iron deficiency and iron poisoning in man.”  Geomedical problems related to aluminum, iron , and manganese.  1992, 65-71.

11. Committee on Medical and Biological Effects of Environmental Pollutants:  Subcommittee on Iron,  Iron.  Baltimore, University Park Press.  1979.

B1 “Simultaneous multi-element determination in vegetable foodstuff and their respective cell fractions by total-reflection X-ray fluorescence (TXRF)” http://ovid.library.upenn.edu/ovidweb/ovi…&ST-3&R 60&totalCit=325&D=mesz&S-13516350.

B2 “Radiochemical neutron activation analysis of Fe, Co, Zn, Sb, and Se in biomedical and environmental samples” http://ovid.library.upenn.edu/ovidweb/ovi…&ST=3&R=37&totalCit=37&totalCit=325&D=mesz&S=13516350

B3 “Electrothermal atomic absorption spectrometric determination of cadmium, copper, iron, lead,and selenium in fruit slurry: analytical application of nutritional and toxicological quality control”         http://ovid.library.upenn.edu/ovidweb/ovi…&ST=3&R=23&totalCit=325&D=mesz&S= mesz&S=13516350