Heat of Combustion of Natural Oils

 W3 

March 21, 1999

Joseph Smith

Michelle Lieberman

Laura Fung

Sabir Rassiwalla

Abstract

The purpose of this experiment was to determine whether the Parr bomb can be used to obtain accurate heat of combustion values for animal oils as well as vegetable oils.  Prior to actual experimentation, the project was outlined in order to maximize time efficiency and minimize human error.  Standardization of the bomb and calorimeter using benzoic acid was completed each of the three lab days.  From a total of five benzoic acid trials, the energy equivalent of the bomb was 2390.75  147.50 Cal/g.  The large 95 % confidence interval of 147.50 Cal/g is due to the benzoic acid trials being done on three different days.

Three natural oils were chosen by the group to be analyzed that were produced by Sigma Chemical Company; one vegetable oil and two animal oils (Olive Oil, Lard Oil, and Fish Oil).  In the first lab session, four Olive Oil samples were combusted and calculated to have an average of 9481.06  67.08 Cal/g.  The data for Olive Oil was both accurate and precise.  The accuracy was measured by a “t” test of significance in which the Te value was .998, which is indeed less than the t value of 3.18.  Precision was measured with a 95% confidence limit.  In the second lab session, Lard Oil was combusted and found to have an average of 9416.88  86.20 Cal/g.  From the test of significance, the Te of 1.28 was less than the t (using n-1 degrees of freedom) of 2.78.  Hence, the bomb gives accurate and precise measurements for Lard Oil.  The third and final lab session was used for Fish Oil.  The average of four Fish Oil trials was calculated to be 9070.17  39.30 Cal/g.  Although the data for Fish Oil was precise, it was not accurate with our predicted value of 9156 Cal/g.  The Te value was 6.98 and the t value was 3.18.  This was caused by improper treatment of the Fish Oil as it was left in an open tray causing oxidation of the components and therefore affecting the heat of combustion.  In conclusion, the Parr bomb can be used to obtain accurate and precise heat of combustion values for animal oils as well as vegetable oils.

Table of Contents

Abstract	2

Table of Contents	4

Background	5

Materials	7

Methods	9

Results	11

Discussion	16

Appendix	26

References	30

Background

Bomb Calorimetry is the scientific process used to determine the heat of combustion (calorific value) of an organic substance.  Essentially, the heat of combustion is the energy stored in the atomic bonds of a specific sample.  In this experiment, a sample of olive (Olea europaea sativa), lard (Sus scrofa), or menhaden fish (Brevoortia tyrannus) oil (weighing no more than one gram) was placed in a Parr bomb calorimeter.  The pressure of oxygen gas caused a reaction to occur between the oxygen and the sample (consisting of carbon, hydrogen, and oxygen).  The carbon and hydrogen in the sample mixed with the oxygen gas to form CO2 and H2O respectively.

As this reaction occurs, the pressure in the bomb increases as the liquid sample turns to a gas.  Because the reaction occurs in an environment of constant volume the temperature in the bomb increases as well.  By recording the temperature change with time, accurate heat of combustion values were obtained.

Using the73rd edition of the Handbook of Chemistry and Physics and the Merck Index, the percent composition of the fatty acids that comprised each of the oils was found.  To calculate the total heat of combustion of each oil, the heat of combustion of each fatty acid contained in the oil was multiplied by its compositional percentage.   The resulting partial heat of combustion (HOC) values were then summed.  The predicted heat of combustion values for each of the oils are listed in Table 1 below.

Table 1- Predicted Heat of Combustion values

| Oil | Predicted HOC |
| --- | --- |
| Olive | -9501.89 |
| Fish | -9156.34 |
| Lard | -9456.62 |

For all of the heat of combustion values of the different fatty acids, data was obtained from the NIST webbook.  The three exceptions, which could not be found in either the NIST webbook or the Handbook of Chemistry and Physicsiv, were C14 monoethanioc acid, C20 and C22 polyethanoic acid.  For these exceptions, W3 created its own formula to calculate their heat of combustions.  This formula is shown below.

Formula 1	Heat of Combustion = 158.25 (Number of Carbons)						         - 37.75 (Number of Double Bonds) 					         - 113.00 (Per Acid Group)

A statistical analysis was performed to compare the values for the HOC of some standard fatty acids versus the predicted value of the HOC using the W3 formula (see appendix for tables of t-test with an alpha level of .05 and comparison chart).  The tests proved that the W3 formula came within 3 kcal/g for each fatty acid compared.

Finally, it should be noted that the standard equations used to calculate the heat of combustion of a sample with a measured temperature change and all necessary thermochemical corrections (for using the Parr bomb) can be found in the laboratory manual.

Materials

The main objective of this project is to obtain accurate and precise heat of combustion values for animal oils as well as vegetable oils.  In this experiment a comparison will also be made to determine whether the Parr bomb is just as accurate and precise with liquid samples as it is with solid pellets.  In order to achieve the highest degree of accuracy and precision, the oils were chosen carefully.  Three oils were purchased and tested over a three-week period; Sigma Olive Oil, Lard Oil, and Fish (Menhaden) Oil.  Using Sigma products assured purity and quality of the oils, and this quality is guaranteed (i.e. the quality of the product can be verified using the batch number, if inaccurate results are found).  Using only three different oils allowed more time for multiple trials per oil.

An estimated heat of combustion was calculated for each oil, so that the accuracy of the experimentally measured values can be determined.  Using the Handbook of Chemistry and Physics, the percent composition of each of the oils was found, and this was used to calculate an estimated heat of combustion for the oils.  The percent composition of each oil is represented in pie chart form (see Figure 1,2,3 below).

Figure 1 - Percent Composition of Olive Oil

Figure 2 - Percent Composition of Lard Oil

Figure 3 - Percent Composition of Fish Oil

Once the percent composition of each oil was determined, an estimated heat of combustion value for each of the oils could be calculated.  This calculation was explained in the background section of the report.

Aside from the different oils being used in the experiment, the rest of the materials were standard and can be found listed in the laboratory manual.  It is important to note, however, that because oils are being combusted, a fine cotton fiber will be used to help ignite the oil.

Methods

Once again it is important to mention that the methods chosen are designed to make the experiment flow as smoothly as possible, as well as to assure accuracy in results.  The procedure for setting up the calorimeter is clearly outlined in the lab manual, but some additional procedures were added.

First, on each day of experimentation, the calorimeter was standardized twice with Benzoic Acid as described in the laboratory manual(after realizing that one trial was not sufficient the first week of experimentation, two trials were used on subsequent weeks).  Next, each oil sample was weighed directly in the combustion capsule, by placing it on the balance, zeroing the balance, and pipetting in the approximate amount of oil needed.  Since the samples used were approximately 1 g, calculations from the known density were made to estimate the amount of liquid that needs to be added to get the desired weight.

Table 1 - Calculated Volume for 1 g Sample

| Oil | Density | Desired Weight | Volume needed |
| --- | --- | --- | --- |
| Lard | 0.915 | 1g | 1.093mL |
| Fish | 0.93 | 1g | 1.075mL |
| Olive | 0.91 | 1g | 1.099mL |

Once the combustion capsule was placed in the bomb, the loop of the fuse wire was placed just above the surface of the sample, but not submerged in the liquid.  One end of a short piece of fine cotton thread was positioned over the fuse loop with the other end touching the liquid.

Precautions were taken when testing heavy oils because the intense heat developed during combustion may damage the interior of the bomb and the electrodes.  Thus the combustion capsule was left  in good condition and slightly tilted in the loop holder so as to direct the flame away from both electrodes (see Figure 4 ).

Oils

Tilted Basket

Figure 4 - Combustion Capsule should be Slightly Tilted (left, standard; right, improved)

When the combustion capsule was tilted, the depth of the liquid increased, while the surface area decreased, facilitating centralized combustion.  At least four trials were performed for each oil sample and the heat of combustion values were calculated using the Parr bomb formula.

The following calculations were done to assess the precision and accuracy of the trials.  For each type of oil, the respective trials were tested for precision by calculating the mean and its 95% confidence interval.   Next the values were compared to the t-critical value of the estimated heat of combustion to see if they were statistically the same (accurate).

The procedures outlined in this proposal, as well as the laboratory manual were carefully followed in order to prevent inconsistencies and errors.

Results

Each day, the calorimeter was standardized with Benzoic Acid twice.  This allowed for a calculation of the energy equivalent of the calorimeter, as well as a determination of overall error of the Parr Bomb itself when measuring the heat of combustion of solid pellets.  Figure 5 below is a graph demonstrating the temperature vs. time curve for the standardization of Benzoic Acid, on the first day of trials.

Figure 5 - Standardization of Benzoic Acid Graph

The energy equivalent of the calorimeter was then determined using the standard value of the heat of combustion of Benzoic Acid, and plugging in the recorded temperature changes with thermochemical correction to find the energy equivalent.

Table 2 below displays the calculations for the energy equivalent of Benzoic Acid.

Table 2 - Benzoic Acid Standardization Data

| Energy Equiv. | mass (g) | Temp. change | ml Titrated | Fuse Wire | HOC Benzoic |
| --- | --- | --- | --- | --- | --- |
| -2391.933 | 0.9996 | -2.65152 | 7.6 | 21.85 | 6315.335 |
| -2256.955 | 0.992 | -2.78653 | 7.7 | 16.56 | 6315.335 |
| -2289.498 | 1.0032 | -2.7772 | 7.9 | 14.95 | 6315.335 |
| -2529.427 | 1.0057 | -2.520675 | 8.2 | 16.33 | 6315.335 |
| -2485.932 | 0.9998 | -2.549375 | 7.4 | 16.1 | 6315.335 |

The average energy equivalent was calculated to be -2390.749  147.5 Cal/g.

Each day, following the standardization of Benzoic Acid, 4 to 5 trials were done for one of the oils.  The next series of graphs display the temperature vs. time curves for the olive, lard and fish oils.

The first day, olive oil was tested.  Below in figure 6 is a temperature vs. time curve for one trial of the olive oil.

Figure 6  - Trial 1 Olive Oil Graph

Four trials were conducted using Olive Oil and the results of each trial are displayed in

Table 3 below.

Table 3 - Olive Oil Results

| E equiv. | mass | temp | mL | Fuse Wire | HOC |
| --- | --- | --- | --- | --- | --- |
| Cal/g | (g) | (C) | Titrated | (cm*2.3) | Cal/g |
| -2390.75 | 0.9992 | -4.006 | 25.3 | 17.48 | 9542.194 |
| -2390.75 | 0.9984 | -3.96475 | 24.95 | 22.08 | 9446.807 |
| -2390.75 | 1.0001 | -3.97788 | 21.4 | 14.72 | 9473.045 |
| -2390.75 | 1.0008 | -3.98008 | 27.9 | 17.71 | 9462.18 |

The average value for the heat of combustion was calculated in Cal/g. Average values were obtained from all four trials and also from just the last three values.  The average of all four trials was 9481.06  67.08 Cal/g.  The average for just the last three trials (discounting the first value which was much larger in comparison to the other 3) was 9460.68  32.75.  These values were compared to the predicted value of 9501.89  2 Cal/g (obtained using the w3 formula and standard heats of combustion of the constituents of the oils) using a t-test.  The value of t found for the data using n-1 degrees of freedom was 3.18 for all four trials, and 4.30 for the last three trials.  Te was then calculated and its value compared to the t values.  Te was calculated to be 0.988 and 1.258 (for all four trials, then three trials, respectively).

The following week, data was collected for five trials with Lard Oil.  A temperature versus time graph is shown in figure 7 below for one of the trials.

Figure 7 - Trial 2 Lard Oil graph

Five trials were done with the Lard Oil.  The results of these trials is displayed in Table 4 below.

Table 4 - Data for Lard Oil

| Ener. Equiv | mass | Temp | Titrated | Fuse Wire | Heat of Combustion |
| --- | --- | --- | --- | --- | --- |
| Cal/g | (g) | (C) | (mL) | (cm) | Cal/g |
| -2390.75 | 0.9997 | -3.979 | 13.9 | 17.02 | 9484.811 |
| -2390.75 | 1 | -3.933 | 14.7 | 16.1 | 9370.820 |
| -2390.75 | 1 | -3.987 | 13.9 | 17.71 | 9500.258 |
| -2390.75 | 1 | -3.926 | 0.9 | 16.1 | 9369.080 |
| -2390.75 | 0.9998 | -3.928 | 16.5 | 17.02 | 9359.405 |

The average value for the heat of combustion for Lard Oil was calculated and found to be 9416.88  86.20 Cal/g.  A t-test was then used to compare this average value to the predicted value of 9456.62  2 Cal/g.  The t value calculated with n-1 degrees of freedom was 2.78, as compared to a value of te for the measured data, of 1.28.

The last day of experimentation, data for the Fish Oil was collected.   A sample graph for the heat of combustion of the fish oil is displayed in figure 8 below.

Figure 8- Fish Oil Graph

The results for all 4 trials are displayed in Table 5 below.

Table 5 - Fish Oil Data

| Energy Equivalent | mass | Temp | Titration | Fuse Wire | Heat of Combustion |
| --- | --- | --- | --- | --- | --- |
| Cal/g | (g) | (C) | (mL) | (cm) | Cal/g |
| -2390.75 | 1 | -3.81616 | 13.8 | 15.87 | 9093.81 |
| -2390.75 | 0.9997 | -3.8088 | 13.6 | 12.42 | 9082.59 |
| -2390.75 | 1.0001 | -3.80332 | 14.1 | 10.35 | 9067.43 |
| -2390.75 | 0.9998 | -3.79143 | 13.2 | 16.1 | 9036.86 |

The average value for the heat of combustion was found to be 9070.17  39.30 Cal/g.  The average for just the first three values was also calculated and found to be 9081.28  32.89 Cal/g.  Both values were compared to the predicted value of -9156.34  2 Cal/g using a t test.  The values of t (using n-1 degrees of freedom) of 3.18 for all the data and 4.30 for just the first 3 trials were compared to the values of te for the measured data.  Te was calculated to be 6.98 for all the data and 9.82 for just the first 3 trials.

Discussion

The purpose of this experiment was to obtain accurate and precise results for the heat of combustion of not just vegetable oils, but also animal oils.  Once the results for the oils have been analyzed a comparison can then be made between the data for the oils and the standardization of benzoic acid.  It can then be determined whether the Parr bomb is just as good if not better at determining accurate and precise heat of combustion values for oil samples versus solid samples.

A confidence limit of 95% was used in order to analyze the precision of the experimental values.   The average heat of combustion of the Olive Oil was calculated to be 9481.06  67.08 Cal/g.  In observing the data for the Olive Oil, it can be seen that our values are indeed precise, as they fall within the 95 % confidence limits of 67.08 Cal/g.  Looking at the data for olive oil (Table 3) it was observed that the last three trials seemed to be closer to one another than the first trial (i.e. the first trial might be an outlying point).  When the average was taken without the first Olive Oil trial, the data still fell within the 95% confidence limits with an interval of  32.75 Cal/g.  Because the data fits within the 95% confidence limits, with and without the outlying point included, it can be concluded that the Parr bomb can obtain precise results with a vegetable oil.

For Lard Oil, precision was measured again by a 95 % confidence interval, which was found to be 86.20 Cal/g.  All of the data again fell within the confidence limits, however the measured values for the heat of combustion of Lard Oil were not as precise as the Olive Oil.  As can be seen from the HOC values of Lard Oil in Table 4, the third trial resulted in a heat of combustion that was only 3 Cal/g away from the maximum limit of the 95% confidence interval (the maximum being 9416.88 + 86.20 Cal/g = 9503.08 Cal/g).  Despite the fact that some of the points were far from the average, it can still be concluded that the calorimeter can be used to obtain relatively precise values for animal oils.

The Fish Oil data was also precise.  The average value from all four trials was 9070.17 Cal/g and by using a 95 % confidence limit, it was observed that our data would fall within  39.30 Cal/g of the mean 95 % of the time if infinite trials were taken.  It was observed that the first three trials were closer together than the last trial, so the mean and its 95% confidence limit was calculated for just those three trials as well.  Not only did all three trials fit within the confidence limit of  32.89 Cal/g, but it is also clear to see that these values are statistically closer together, and therefore more precise.

In order to analyze whether the Parr Bomb was more precise in determining the heat of combustion of animal oils versus vegetable oils, the confidence intervals for each of the oils was measured as a percent of the average value.  By measuring the intervals as a percentage, the values for each oil can be compared with one another.  The olive oil had a confidence interval of .71%, the lard oil had an interval of .92%, and the fish oil had a confidence interval of .43%.   Therefore, it can be concluded that the Parr bomb gives just as precise measurements for vegetable oils as animal oils.  It can also be concluded that the Parr Bomb has a level of precision that is within 1% over the range of trials.

In order to compare the precision of the Parr bomb with solids (benzoic acid pellets) versus the oils, the confidence interval for the Parr bomb can also be measured as a percent of the heat of combustion of benzoic acid.  The interval for the benzoic acid was 2.3%, which is much higher than the other confidence intervals.  It should be noted, however, that the benzoic acid trials, though there were five done, were all done on different days.  Observing the values taken each day, it is clear to see that they are quite close together (since only one trial was done the first week, this comparison can only be done for the second and third week).  For the second week, there was only a difference of  32.54 Cal/g between the two values, and for the third week, only a difference of 43.49 Cal/g between the two values.  If we measure these differences as a percentage of the total heat of combustion of benzoic acid, they come out to be .52 % for day two and .69% for day 3.  After talking with technical services at Parr Instrument Company, it was learned that the room temperature can effect the values obtained for the energy equivalent when standardizing with benzoic acid.  The fact that the temperature in the room varied day to day (the first day it was warm and humid, the second day cooler) may explain why such variations in the energy equivalent of the calorimeter were found.  When looking at the overall differences in values it looks as though the Parr bomb was more precise with animal and vegetable oils than the benzoic acid.  By taking a closer look, however, at the day to day precision (because other factors such as temperature might have affected the standardization), it is clear to see that the Parr bomb is just as precise in measuring the heat of combustion of the oils as it is with the solid pellets (benzoic acid).

In order to measure the accuracy of our heat of combustion values, a “t” value test of significance was used (using n-1 degrees of freedom). For the Olive Oil, our predicted value using the method described in the methods section was 9501.89 Cal/g and our average measured value was 9481.06  67.08 Cal/g.  The value of t found for the data was 3.18 for all four trials.  Te was then calculated and its value compared to the t value.  Te was calculated to be 0.988, which proves that our data is accurate with the value we predicted within our 95 % confidence interval.  As was stated earlier, it was observed that the last three trials were closer together and the first trial seemed to be an outlying point.  It makes sense that the first trial might not be as accurate as the others due to unfamiliar procedure (as it was the first time w3 had done an oil trial).  Looking at the three trials that were closer together, the results are still accurate, as the Te value was 1.25, as compared to a t value of 4.3.  Therefore, it can be concluded that the Parr bomb can be used to obtain accurate values for vegetable oils.

The predicted value for Lard Oil was 9456.62 Cal/g and the average heat of combustion measured from all five trials was 9416.88  86.20 Cal/g.  Again, the t-test of significance was used to determine the accuracy of our measurements.  The value of t for the data was 2.78 (using n-1 degrees of freedom) and was then compared to the value of Te which was calculated to be 1.28.  Because the value of Te is less than that of t, our data is proven accurate to that of our predicted value within a 95 % confidence limit.  Despite the fact that the Te value was less than the t value, it was observed that the values lacked consistency.  The graph in Figure 9 below demonstrates the unpredictability of the measurements.

Figure 9- Lard Oil Trials

It is clear from the above graph that while 2 of the values were very close to the predicted value, three were not significantly close, and brought the overall average down.  This graph demonstrates that the predicted value was never obtained during experimentation, and the average value is therefore not a good representation of the actual results of the experiment.  In order to determine whether the Parr bomb is an effective instrument for obtaining accurate heat of combustion results, more trials would need to be done.

The average heat of combustion measured for Fish Oil was 9070.17  39.30 Cal/g.  The predicted value for this oil was 9156.34 Cal/g.  Although, as mentioned above, the data for fish oil was very precise, the data was not accurate within our 95 % confidence limit.

While executing the experiment we found that the Fish oil had been refrigerated instead of stored at room temperature.  This caused a separation of the solid phase and liquid phase of the fatty acids in the fish oil.  To allow the two phases to mix again we set the fish oil out at room temperature for an extended period of time.  Upon contacting Sigma we found out that fish oil is readily oxidized when open to the environment. Knowing that the oxidized oil has different molecular components and therefore different calorimetric properties, this could account for the discrepancy.  This theory was also reinforced when examining the each individual trial as shown in Table 5 in the results section.  The trials are listed in order of execution, and between each trial there was an approximate down time of one hour while assembly, cleaning, etc. of the Parr bomb was completed.  Obviously the fourth fish oil trial will have been sitting longer in the air and more oxidized than the first (this also explains the odors which came from the oil).  As seen from the data the first fish oil trial was 9093.81 cal/g, which is much closer to the predicted value of 9156.34 cal/g.  The graph below depicts the change in HOC with each trial, using trial as a correlate to time.

Figure 10 - Fish Oil HOCs vs. trial number (as the fish oil continues to oxidize)

Since the x-axis represents the order of the trials and not a definite time period we cannot fit a trendline to the data and correlate the x-axis and y-axis.  However by objectively looking at the graph and assuming that we waited about an hour to do the first trial, we can roughly see that by extrapolating backwards around the 0 hour the HOC may have been above 9100cal/g and much closer to our predicted value.  Also keep in mind to really analyze the graph below one must evaluate rate of reactions, since that is what the graph really represents.  However we do not have the measurements and tools necessary to do this (i.e. time, temperature of day, composition of air on day, etc.)

Another source of error comes in the calculation of the predicted heat of combustion of the fish oil. There were two fatty acids, C20 polyethanoic and C22 polyethanoic, which we could not get confirmation on their exact structure (though the Merk Index, Bellstein Database, NIST webbook, CRC 73rd edition Handbook of Chemistry and Physics, and multiple organic chemistry professors were consulted).  A slight error in predicted structure greatly affects the w3 formula (as it depends on the number of double bonds and the number of carbons).  In order to obtain accurate results, the experiment should be re-done with better procedure, and a more accurate predicted value for the heat of combustion.  Due to these factors, an evaluation of the Parr bomb’s effectiveness at measuring accurate heat of combustion values for fish oil cannot be made.

Overall the data for the heat of combustion of olive, lard and fish oils demonstrates that accurate and precise values can be measured using the Parr bomb.  Since the largest percent error found for any of the oils is .92% for the lard oil, we can assure that the Parr bomb will measure animal oils to at the most 1% error.  When comparing the measurement of solid samples, such as benzoic acid, to vegetable oils, the error in using vegetable oils compared to solid samples is a little less than two times greater.  The percent error in benzoic acid is .52% for day two and .69% for day three; the average of these two values yields approximately .61%.  Since the maximum percent error was found in lard oil (.92%), there is approximately a two-fold difference.  However, keep in mind that since the magnitudes of these errors is so small a two-fold difference is still less than the 1% error that we aimed for.

The above statements deal with precision, but the accuracy of the Parr bomb must also be evaluated.   As mentioned above, we were able to prove the accuracy of the Parr bomb for two of the three oils using a .05 alpha level.  Due to our time constraints we were only able to use two animal oil samples, fish and lard.  As explained above our fish oil did not agree with our predicted values while our lard oil did.  Since we are only able to speculate of our errors with the fish oil, a full analysis of the Parr bomb’s accuracy for animal oils cannot be made.  More trials need to be done to fully assess how accurate the Parr bomb is for animal oils versus vegetable oil.  However, a comparison can be made between the benzoic acid and the olive and lard oils to assess the Parr bomb’s accuracy.  Since results were obtained for the olive and fish oils that had less than 1% error, it can be concluded that the Parr bomb is accurate to within 1% for oils.  In order to evaluate the Parr bomb’s accuracy with the benzoic acid, a reverse calculation was done to see if putting in the average energy equivalent that was measured could yield the correct value for the heat of combustion of benzoic acid (6315.335 Cal/g).  The values found for the heat of combustion when the reverse calculation was done are shown below, along with their percent difference from the actual heat of combustion of benzoic acid.

Table 6 – Heat of Combustion with Reverse Calculation

| Heat of Combustion | % Difference |
| --- | --- |
| 6312.193 | 0.05% |
| 6691.163 | 5.95% |
| 6595.632 | 4.44% |
| 5967.755 | 5.50% |
| 6072.630 | 3.84% |
| Average HOC | Average % Diff |
| 6327.875 | 3.96% |

Since the average % difference is 3.96%, it is clear to see that the Parr bomb appears to be more accurate for the oils than the solid benzoic acid pellets.  However, more trials with benzoic acid should have been done each day to actually prove that the bomb is not as accurate with solids (3.96%) versus oils (less than 1%).

Overall, it is clear to see that the Parr bomb could be used to obtain both accurate and precise heat of combustion values for animal and vegetable oils (within 1%).  While it seems that the Parr bomb is just as precise whether the sample is an oil or solid pellet, more trials should be done to confirm this point.  Also, in order to determine whether or not the Parr bomb is more accurate with animal versus vegetable oils, and solids versus liquids, more trials should be done in order to support or reject the observations of this experiment.

Appendix

Table A – Sample calculation for olive oil’s heat of combustion

| Olive Oil | %composition | HOC (Cal/g) | Partial HOC |
| --- | --- | --- | --- |
| Palmitc | 6.9 | -9297.25 | -641.51 |
| Stearic | 2.3 | -9483.97 | -218.13 |
| Arachidic | 0.1 | -9616.07 | -9.62 |
| Oleic | 84.4 | -9550.91 | -8060.97 |
| Linoleic | 4.6 | -9417.99 | -433.23 |
|  |  | Predicted HOC | -9363.45 +/- 2 Cal/g |

Table B – Comparison of W3 formula versus actual HOC values

| Name | # of Carbons | # of double bonds | Actual Heat of Comb. | PredictedW3 value |
| --- | --- | --- | --- | --- |
| Linoleic acid | 18 | 3 |  |  |
| Palmitic | 16 | 1 | 2384 | 2381.25 |
|  |  |  | 2384 | 2384 |
| Steric Acid | 18 | 1 | 2698 | 2697.75 |
| (all types of ") |  |  | 2696 | 2696 |
| ethyl ester | 20 | 1 | 2994 | 3014.25 |
| stearic acid/ee | 20 | 1 | 3012 | 3014.25 |
| methyl ester | 19 | 1 | 2832.6 | 2856 |
| methyl ester | 19 | 1 | 2859 | 2856 |
| nonadecanoic | 19 | 1 | 2849 | 2856 |
| oleic | 18 | 2 | 2667 | 2660 |
|  |  |  | 2659 | 2660 |
|  |  |  | 2659 | 2660 |
| methyl ester |  |  | 2659 | 2659 |
| palmitoleic | 17 | 2 | 2521 | 2501.75 |
| myristic | 15 | 2 | 2254 | 2185.25 |
| Palmitic meth | 17 |  | 2550 | 2577.25 |
|  |  | 1 | 2415 | 2415 |
| Linoleic methest | 19 | 3 | 2794 | 2780.5 |
| Linolenic | 19 | 4 | 2750 | 2742.75 |
| Stearic | 19 | 1 | 2859 | 2856 |
| Oleic | 19 | 2 | 2828 | 2818.25 |

Table C - Statistical Analysis of W3 predicted values versus actual values

| t-Test: Paired Two Sample for Means |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Actual | W3 |
|  | Heat of Comb. | Predicted |
| Mean | 2682.07619 | 2679.583333 |
| Variance | 41782.2619 | 45845.50833 |
| Observations | 21 | 21 |
| Pearson Correlation | 0.997005573 |  |
| Hypothesized Mean Difference | 0 |  |
| df | 20 |  |
| t Stat | 0.605142062 |  |
| P(T<=t) one-tail | 0.275944909 |  |
| t Critical one-tail | 1.724718004 |  |
| P(T<=t) two-tail | 0.551889817 |  |
| t Critical two-tail | 2.085962478 |  |

Note: The null hypothesis was that there was no difference between the actual and W3 predicted means.  The probability of the two tail test needs to be greater than .95 (due to the alpha level of .05) for there to be a statistical difference between the two means.  Thus the value of .55 demonstrates that the null hypothesis cannot be rejected.

Figure A – Olive Oil Trials

Figure B – Lard Oil Trials

Figure C – Fish Oil Trials

Figure D – Comparison of Heat of Combustion Values for the Oils

References

Page 7-29, Handbook of Chemistry and Physics, 73rd ed. David R. Lide. CRC Press, Inc.

The Merck Index, 12th ed. Susan Budavari, et Al. Merck & Co. Inc.

webbook.nist.gov/cgi/cbook.cgi

Operating the 1108 Oxygen Bomb. Parr Operating Instructions for the 1108 Oxygen Combustion Bomb.

Page 7-29, The Handbook of Chemistry and Physics, 73rd edition. CRC

“Apparatus, Equipment and Reagents” of Calorimetry: Heat of Combustion of Sucrose.  BE 210 Laboratory Manual. Litt.

Page 6 “Liquid Samples” of Samples and Sample Holders. Parr Operating Instructions for the 1108 Oxygen Combustion Bomb.

Operating the 1108 Oxygen Bomb. Parr Operating Instructions for the 1108 Oxygen Combustion Bomb.

Sigma Chemical Company. Biochemicals Organic Compounds and Diagnostic Reagents. 1996.

Page 7 “Heavy Oils” of Samples and Sample Holders. Parr Operating Instructions for the 1108 Oxygen Combustion Bomb.

Heat of Combustion of Oils, R4 Spring 1998