# PROJECT FINAL REPORT COVER PAGE

GROUP NUMBER R1

PROJECT TITLE:  Determination of the Heat of Combustion of Tryptone

DATE SUBMITTED:  May 7, 2001

# ROLE ASSIGNMENTS

ROLE                                                     GROUP MEMBER

FACILITATOR…………………………Nina Teng

TIME & TASK KEEPER………………Geeta Bhargave

SCRIBE……………………………….. Frank Casey

PRESENTER…………………………..Fahad Haque

COMMUNICATOR…….……………..Briseyda Morales

# SUMMARY OF PROJECT

The objective of this experiment was to determine the heat of combustion of tryptone precisely and accurately using the Parr Bomb Calorimeter, and to observe whether precision and accuracy vary with the mass size of the pellet.  In order to determine the energy equivalent, W, of the calorimeter, the combustion of benzoic acid was carried out three times.  The mean W value for the three trials was found to be 2471.1 cal/C with a 95% confidence interval of  37.3 cal/C.   Twelve combustion trials using tryptone were performed with three different mass sizes:  0.5g, 1.0g, and 1.5g.  The mean heat of combustion was found to be 4894.7 cal/g with a 95% confidence interval of  92.1 cal/g.  Although it was determined using t-tests that the accuracy between the three masses were not statistically different, the precision increased with an increase in the mass of the pellet.  The heat of combustion of tryptone that was calculated using the composition was found to be 3935 cal/g with an uncertainty of  2.118 cal/g.  The mean experimental value differed from the calculated value by 24%.

Objectives

The determination of the heat of combustion of tryptone for caloric purposes is pertinent to the field of biological and chemical research sciences, since tryptone is one of the principal ingredients for nutrition included in bacteria and yeast growth mediums. The objective of the experiment was to determine the heat of combustion of the compound tryptone using a non-adiabatic bomb calorimeter standardized with benzoic acid.  By taking into account the amino acids that make up the composition of tryptone, its calculated value of heat of combustion was also determined to compare the accuracy of the experimentally obtained value with respect to the calculated value.  In addition, comparisons among the accuracy and precision of the experimentally obtained heat of combustion values of tryptone for three different mass sizes of 0.5 gram, 1.0 gram, and 1.5 gram were also considered.

Background

The heat of combustion of a substance is the amount of energy released when an organic substance is burned completely in oxygen.  This value is useful in determining the maximum amount of energy that microbials can utilize through a bioenergetic reaction with a substance. Microbials convert the chemical energy found in the atomic bonds of tryptone to energy for their own use. The organisms then use this energy through biochemical reactions to create ATP and NADH, thus sustaining survival of the microorganisms.1

A process called bomb calorimetry is used to determine the heat of the combustion of a substance.  This measurement is taken by using a bomb calorimeter, a thermodynamically insulated container. A weighed sample of a substance, placed in a bomb calorimeter, is surrounded with a specific amount of water.  Pressurizing the bomb with oxygen creates a reaction between the oxygen and the substance.  Using the temperature increase that occurs due to the reaction, the heat of combustion can be calculated2.  All the basic calculations and correction factors can be found in the Parr manual3.

Tryptone is a pancreatic digest of the milk protein, casein.  The substance is used as a moderately rich, general-purpose media for microbiological cultures.  As a rich source of nitrogen, tryptone is used for isolating bacteria and fungi. Additionally, the lack of carbohydrates in tryptone makes it useful in distinguishing bacteria based on their ability to ferment certain carbohydrates4.

As cited in the Difco Manual, tryptone is composed of 80.18% amino acids, 7.70% carbohydrates, 6.00% inorganics, and 6.80% ash.  Within the inorganics and amino acids is included 13.00% nitrogen.  These values were obtained from the original maker of tryptone, and not from Fisher Scientific, the company that made the tryptone used in the experiment.  Fisher Scientific was contacted, but failed to provide their composition of tryptone.  Other common names for tryptone include Peptone C, Peptone 50, and Tryptone T.

# Theory and Methods of Calculations

Before analyzing data, the heat of combustion of tryptone was calculated by multiplying the percent composition of each substance by that substance's heat of combustion and then summing all of the results.  The values for the amino acids and carbohydrates were found using the NIST webbook5.  This general recipe was found in the Difco manual, the original manufacturer of tryptone.  These calculations resulted in a textbook value of 3935 cal/g with an uncertainty of  2.118 cal/g.  Because this general recipe did not come from Fisher, it only was used as a base value to compare with the values for the heat of combustion obtained for tryptone in this experiment.

# Materials, Apparatus, Methods

The materials and apparatus used were the same as those listed in the manual, with the exception of the replacement of tryptone for sucrose. Among those apparatus used were the Parr Instrument Model 1341 Oxygen Bomb Calorimeter and the Parr Instrument Model 1108 Oxygen Combustion Bomb.

The experiment was subdivided in two parts: determination of the energy equivalent of the calorimeter (standardization) and the determination of tryptone’s heat of combustion with varying mass.  In both cases, the procedure was the same, the difference being the sample that was being analyzed.  Since initial temperature has an influence on the magnitude of both the energy equivalent and the heat of combustion of tryptone, the initial temperature was fixed for all trials between 24°C and 25°C.  For the standardization of the bomb calorimeter, the procedure outlined in the manual was followed, using approximately 1.00g of benzoic acid. The remaining product was then titrated with 0.0354N sodium carbonate. Three independent trials were performed. Using the relevant data, the energy equivalent of the calorimeter was calculated.

In order to judge whether or not mass affects precision and/or accuracy, the heat of combustion of tryptone was measured for three different masses, with four independent trials per mass. The three masses were approximately 0.50g, 1.00g, and 1.50g. All the tryptone trials failed to burn completely, due to the ash in its composition. This ash was weighed, its percentage contribution calculated and compared to the actual value.

## Calculations

The experimental values of tryptone were calculated using the following equations:

Hg = tW-e1-e2-e3			t  = ta-tc-r1(b-a)-r2(c-b)

m

Hg = heat of combustion	ta = initial temperature

t = corrected temperature rise	tc = final temperature

W = energy equivalent of the calorimeter	r1 = slope of pre-period

e1 = nitric acid correction (mL of .0709 alkali solution)	r2 = slope of post-period

e2 = sulfuric acid correction (13.7 * % sulfur * mass)	a = time of firing

e3 = fuse wire correction (2.3 * cm of wire burned)	b = time when temperature reaches 60% rise

m = mass of tryptone pellet	c = time of peak

Figure 1:  Typical Time Versus Temperature Data

The above graph represents the temperature versus time data collected for the first standardization trial of benzene.  The points marked A, B, and C corresponds to those in the previous heat of combustion equations.  The pre-period, rise period, and post-period, separated by points A and C, are also marked on the graph.  The slope of the pre-period and post-period were taken and used as r1 and r2 respectively.  Similar data was collected for the other two benzene standardization trials and all twelve tryptone trials.

|  | Trial 1 | Trial 2 | Trial 3 |
| --- | --- | --- | --- |
| Calculated W | 2471.2  22.54cal/C | 2486.1  15.82cal/C | 2456.0 8.74 cal/C |
| Average W | 2471.1 cal/C |  |  |
| 95% Confidence Limit |  37.3 cal/C |  |  |
| Relative Standard Deviation | 0.61% |  |  |
| Manufacture's W | 2426 cal/C |  |  |
| Percent Difference | 1.86% |  |  |

Table 1: Calibration Data for Bomb Calorimeter

This calibration data was calculated by manipulating this equation to solve for W, the energy equivalent of the calorimeter.  The values for W for the three calibration trials were then averaged to yield a value of 247.1 cal/ C, with a 95% confidence value of  37.3 cal/gram.  This value for W was used for all further calculations.

|  | Trial 1 | Trial 2 | Trial 3 | Trial 4 | Average | 95% Con. | Rel. St. Dev. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0.5 Grams | 4853.5 | 5270.3 | 4631.6 | 4841.3 | 4899.2 cal/g |  425.8 | 5.46% |
| 1.0 Grams | 4854.1 | 4928.4 | 4863.2 | 4802.6 | 4862.0 cal/g |  82.2 | 1.06% |
| 1.5 Grams | 4915.4 | 4944.0 | 4911.6 | 4920.8 | 4923.0 cal/g |  23.1 | 0.29% |
| Total |  |  |  |  | 4894.7 cal/g |  92.1 | 2.96% |

Table 2: Tryptone Heat of Combustion Results (All in calories/gram)

Using the original equations, the heat of combustion was measured for each trial.  The data for each pellet size was then analyzed by averaging all four results for each pellet size.  The 95% confidence values and relative standard deviation were then calculated for all three sets of data.

Table 3: T-Test Between Different Mass Categories Using Heat of Combustion Values

Using three t-tests assuming no variance, each set of data was compared to the others to observe whether they are statistically the same.  As seen above the heat of combustion values are not significantly different for tryptone at different mass sizes, because T statistical is less than T critical.

Figure 2: Percentage of Ash Retrieved for Each Mass Size (With 95% Confidence Limits)

For each tryptone trial the mass of ash remaining was recorded and its weight percentage was calculated.  The above graph shows the average ash weight percentage for each mass group with the 95% confidence values included.

Figure 3: Comparison between Pellet Size and Nitric Acid Formation

For each trial the volume of alkali solution required to titrate the nitric acid formed during the combustion was also recorded.  In the above graph this volume required was plotted against pellet size in grams to observe their relationship.  It was observed that the data have a highly linear relationship, with a R2 value of .9957.

| Literature value of sucrose = 3941.30 Cal/g |  |  |  |
| --- | --- | --- | --- |
|  | W Value (cal/C) | Calc Sucrose Hc (cal/g) | % Error |
| Manufacture | 2426 | 3835.01 ± 127.50 | -2.70% |
| Our Results | 2471.1 | 3906.39± 127.50 | -0.88% |

Table 4: Comparison of W values

To analyze the accuracy of the experimental energy equivalent of the bomb calorimeter found during the standardization with benzoic acid, results from a previous experiment were recalculated using the new value for W.  As seen in the table above, the percent error in the sucrose heat of combustion value decreased from –2.70% to –0.88%, when the new experimental W was used in place of the manual value.

| Sources | Range | Effect on Hc |
| --- | --- | --- |
| Measurements of the fuse wire | ± 0.5mm | ±0.115 Cal |
| Mass of benzoic acid & Tryptone | ±0.0001g | ±0.34 Cal |
| Titration of the acids formed | ±0.5 mL | ±0.5 Cal |
| Temperature reading | ±0.005oC | ±12.355 Cal |
| 2000-mL of distilled water | ±1 mL | ±2.47 Cal |
| The total systematic error: | For 0.50g | ± 22.54 Cal |
|  | For 1.00g | ±15.82 Cal |
|  | For 1.50g | 8.74 Cal |

Table 5: Systemic Errors

To analyze the uncertainty of the experiment, systemic errors due to the equipment used and their effect on the heat of combustion were tabulated.  Thus total systemic error lead to an uncertainty of ±22.54 Cal for the 0.5g category, ±15.82 Cal for the 1.00 gram category and ±8.74 Cal for the 1.50g category.

Discussion and Analysis

Several factors were analyzed in determining where discrepancies in the values of the heat of combustion originated.  The factors analyzed ranged from normal systemic error to incomplete combustion of the pellet.

Initially calculations were performed to examine the accuracy of the heat of combustion of sucrose using two different W (energy equivalent) values.  The first W was obtained from the Bomb-Parr manual.  The second was obtained experimentally during the calibration trials, where benzoic acid was burned.   This was done in order to test the validity of the W value that was obtained experimentally.  Both W values were used to calculate the heat of combustion of sucrose.  The data used for comparison came from experiment six (determining the heat of combustion of sucrose) that was performed during course of the year.

Table 4 indicates that the percent error using the manufacturer’s W when finding the heat of combustion for sucrose was -2.7% off from the literature value.  On the other hand, using the experimental W from the calibration trials there was only a -0.88% difference from the literature values of sucrose.  This shows that the W used in these calculations was more accurate than the manufacturer’s value, as would be expected because the manufacturer’s value is generic and does not account for slight differences in the bombs.

T-tests were performed on the samples.  They were conducted between the different mass categories using the heat of combustion values that were obtained for each trial at the various mass sizes, the mass categories being 0.5g, 1.0g, and 1.5g.  These results are summed up in table 3.  From the t-test, it was determined that the heat of combustion values between the different mass categories were statistically equivalent to one another.

Through analysis of the trend line from figure 3, it was concluded that the bomb calorimeter reactions went to completion for all pellet sizes.  This is because the linear regression between the titration volume and pellet size has an R2 value of 0.9957, which shows an extremely high correlation.  The main contribution to nitric acid formation was the amine group (NH3) in the amino acid structures.  Thus if the higher mass sizes did not burn to completion their titration values would not be on the linear trend line, as shown, and the correlation value would be much lower.  From this, it can be concluded that systemic error did not evolve from incomplete combustion of the pellets.

Figure 2 shows the percentages of the ash collected for every mass trial category.  From the graph it is observed that the ash that resulted after each trial has slight variation in percentages.  This resulted from the inexact method of extracting the ash from the crucible at the end of each trial.  The ash was scraped out using a small screwdriver, resulting in small discrepancies in masses.

The sources of systemic errors in this experiment were the measurement of the fuse wire, the mass of benzoic acid, tryptone, the titration of the acids formed, measurement of the 2000 ml of distilled water, and temperature reading. The respective uncertainties due to these errors in the heat of combustion are summed up in Table 5 in the results section.  Total systemic error lead to an uncertainty of ±22.54 Cal for the 0.5g category, ±15.82 Cal for the 1.00g category and ±8.74 Cal for the 1.50g category.

The calculated value for the heat of combustion of 3935.04  2.12 cal/g is about 24% different from the value obtained experimentally which was 4894.7  92.1.  This discrepancy may be due to several factors.  The most likely explanation is that the composition of tryptone outlined in the Difco Manual was not the same composition used by the Fisher brand.  This may be due to the method of processing the tryptone.  It is possible that the tryptone is made by adding a particular unknown ingredient until the desired result is achieved.

Several possible scenarios were calculated to achieve the desired heat of combustion, as shown in Table 6.  They include the addition of all amino acids, the addition of the amino acid with the largest heat of combustion, the addition of carbohydrates, and the addition of tryptophan while decreasing all other amino acids.

|  | all amino acids | amino acid w/ largest Hc | carbs | tryptophan |
| --- | --- | --- | --- | --- |
| Carbohydrates | 0% | 5.00% | 125% | 7.70% |
| Amino Acids | 108% | 95% | 0% | 58% tryptophan, 24% other amino acids |
| Ash | 0% | 0% | 0% | 6% |
| Inorganics | 0% | 0% | 0% | 5% |

Table 6:  Other possible compositional scenarios.

For instance, amino acids may have been added until the tryptone had the desired potency.  In order to have the same calculated heat of combustion as the experimental value with an addition of amino acids while keeping the relative portions of the amino acids the same, 108% amino acids would yield a heat of combustion of 4890.00 cal/g, which is impossible.  Another possibility was calculated by increasing weight percent of the amino acid with the largest heat of combustion value, phenylalanine, until the experimental value was obtained.  As can be seen in the table, this would require 95% amino acids and 5% carbohydrates.  Both of these scenarios are not logical because it was experimentally proven that there is ash in the composition, which would not be accounted for using these compositions.

Another possible explanation may have been more carbohydrates were used.  In order to achieve the experimental heat of combustion with the addition of carbohydrates, 125% carbohydrates would have to be used, which is not possible.  The addition of carbohydrates alone is not enough to achieve the desired heat of combustion.

The final, most logical scenario is depicted in the last column.  This was obtained by increasing the tryptophan content in the amino acids, while decreasing all other amino acids so that the total amino acid weight percent remained the same.  This was done because the Fisher Scientific label on the package of the tryptone indicated that the product was highly enriched with tryptophan.  Additionally, the ash percent was decreased to match the ash percent achieved experimentally.  The calculated heat of combustion for this composition was equivalent to the experimental heat of combustion.

Another possible source of discrepancy is that the specific carbohydrates were not depicted in the Difco manual.  Because tryptone is a digest of casein, a milk protein, the carbohydrate used was probably either dextrose or lactose.  Using either of these yields a heat of combustion value within 1% of the other.  It is unlikely that any other carbohydrates were used.  In addition, most carbohydrates have similar heat of combustion values, and a substitution would not have a significant effect.  Identifying the specific carbohydrate was probably not a source of error in the calculations.

Conclusions

- The combined mean experimental values for the heat of combustion of tryptone for the three different pellet mass sizes was 4894.7 cal/C, which did not lie within 2% accuracy of the calculated value which was 3935.05 cal/C with an uncertainty of  2.12 cal/gram.

- The total precision among the heat of combustion of tryptone values among the twelve experimental trials amounted to a relative standard deviation of 2.96%.

- Although the precision increased with mass size, t-tests showed the average values for each mass size to be statistically equivalent.

References

- Campbell, Reece and Mitchell, 1999. Biology, 5th Edition

- Zumdahl, S., 1993.  Chemistry, 3rd Edition.  D.C. Heath & Co., Ch. 12.

3.   Operating the 1108 Oxygen Bomb. Parr Operating Instructions for the 1108 Oxygen   Combustion Bomb.

- Difco Manual, 11th Edition. 1998.  Difco Laboratories, Division of Becton Dickinson   and Company.  Sparks, Maryland 21152.

- webbook.nist.gov

Appendix

Table A1:  Heat of Combustion of Tryptone Calculation

|  | Weight % | Hc (cal/g) | Partial Hc |
| --- | --- | --- | --- |
| Ash | 6.800 |  |  |
| Carbohydrates |  |  |  |
| Lactose | 7.700 | -3946.480 | -303.879 |
| Amino Acids |  |  |  |
| Alanine | 2.860 | -4349.938 | -124.408 |
| Arginine | 3.030 | 5126.706 | 155.339 |
| Aspartic Acid | 6.110 | -2875.131 | -175.671 |
| Cystine | 0.420 | -4225.135 | -17.746 |
| Glutamic Acid | 17.050 | -3655.781 | -623.311 |
| Glycine | 1.750 | -3101.106 | -54.269 |
| Histidine | 2.020 | -4937.995 | -99.748 |
| Isoleucine | 4.400 | -6520.081 | -286.884 |
| Leucine | 7.110 | -6504.978 | -462.504 |
| Lysine | 6.700 | -6021.684 | -403.453 |
| Methionine | 2.570 | -5709.021 | -146.722 |
| Phenylalanine | 3.710 | -6722.562 | -249.407 |
| Proline | 7.450 | -5701.034 | -424.727 |
| Serine | 4.290 | -3293.653 | -141.298 |
| Threonine | 3.580 | -4182.581 | -149.736 |
| Tryptophan | 0.710 | -6586.691 | -46.766 |
| Tyrosine | 1.420 | -5841.051 | -82.943 |
| Valine | 5.000 | -5938.284 | -296.914 |
| Inorganics |  |  |  |
| Calcium | 0.013 |  |  |
| Chloride | 0.186 |  |  |
| Cobalt | 0.001 |  |  |
| Copper | 0.001 |  |  |
| Iron | 0.001 |  |  |
| Lead | 0.001 |  |  |
| Magnesium | 0.017 |  |  |
| Manganese | 0.001 |  |  |
| Phosphate | 2.669 |  |  |
| Potassium | 0.229 |  |  |
| Sodium | 2.631 |  |  |
| Sulfate | 0.241 |  |  |
| Tin | 0.001 |  |  |
| Zinc | 0.003 |  |  |
| Total Weight % | 100.675 |  |  |
| Total Hc |  |  | -3935.045 |