BE 209 Laboratory Report

Design Project: Building a Polygraph

Group ID: R8

INTRODUCTION:

In criminal investigations, polygraphs have a reputation for being an unreliable source of proving guilt or innocence.  In many states, a polygraph exam is not admissible as evidence in a court of law, and polygraph evidence is widely called into question.  The polygraph exams are designed to monitor physiological signals of the suspect and detect physical arousal, since it is believed that a change in arousal during questioning is indicative of deceit.  The motivation of our experiment is to examine these conceptions about polygraphs by building our own polygraph, administering exams on a number of subjects and interpreting the polygraph data to determine the guilt or innocence of our subjects.  Our objective in this experiment is to build a functional polygraph from a simple bridge circuit useful for detecting physiological changes in a subject when that subject falsely responds to a question.

In polygraph exams, the four basic physiological signals that are monitored for changes are brain waves, galvanic skin response, pulse and respiratory rate.  The physiological responses that we will be testing in our experiment will be respiration rate and breath temperature.  Respiration rate is a common response that is monitored during polygraph testing.1  In addition to monitoring respiration rate, we will also look at the effects of breath temperature in evaluating deceit.  The thermistor apparatus that we will be using is a device that has a resistance proportional to temperature and will provide data output for both of these responses.  Using the Steinhart-Hart equation, we will be able to determine the relationship between the temperature of a subject’s breath and the voltage in the circuit.  The circuit is comprised of a simple bridge using two 22KOhm resistors, a potentiometer and the thermistor.  The bridge is used to reduce the amount of noise in the circuit to a negligible amount.

HYPOTHESIS:

Our hypothesis is that the change in respiration rate, along with a change in the breath temperature, is an accurate indicator of deceit.  The data from the thermistor will give two indicators of deceit, temperature change in the breath and number of breaths per unit time.  The synthesis of this data will indicate the validity of the subject’s responses. We further hypothesize that the guilty knowledge test and stimulation task will be accurate methods for detecting guilt or innocence.

## MATERIALS AND METHODS- THE CIRCUIT:

For this project, we decided to use a simple bridge using the MC7805 voltage regulator chip.  On one arm of the bridge there is a potentiometer and a 22 KOhm resistor in series.  On the other arm of the bridge there is a thermistor and another 22 KOhm resistor in series.  The potentiometer is set to approximately 10 KOhm since the thermistor resistance approximates this value.  We will measure the voltage difference between 2 reference points in the bridge.  The first reference point (A) will be on the first arm of the bridge between the potentiometer and the resistor.  The second reference point (B) will be on the second arm between the thermistor and the resistor.  By comparing the voltage differences between points A and B we can monitor the voltage change, and hence, the change in resistance due to temperature change of the thermistor.  Voltage drop across A and B will be measured by our lie detector Virtual Instrument.  We are using a thermistor to measure respiratory rate because the change in resistance of the thermistor is directly proportional to the change in temperature of the air breathed out.  Measuring the voltage drop across points A and B will show us the change in voltage across the thermistor, which will indicate the change in resistance.  We will use the Steinhart-Hart equation to equate the change in voltage of the thermistor to the change in breath temperature.

The virtual instrument will show the change in voltage with every breath.  Every combination of inhalation and exhalation will show as a cycle on the graph.  From this graph, the respiration rate of the subject can be monitored, and any abnormalities can be easily detected.  The lie detector Virtual Instrument is also equipped with a trigger device that sends a voltage impulse when the trigger is pressed.  This will enable the interrogator to know when each question has been asked by looking at the graph.

There are two possible ways in which the thermistor can be attached; in the first way, the thermistor is attached directly to the skin under the subject’s nose.  In the second way, the wires of the thermistor are held close to the subject’s face, so that the thermistor is hanging under the subject’s nostril.  These two methods are tested in the preliminary data and will enable us to find which attachment yields the most sensitive results.

# METHODS- QUESTIONING:

There are two different modes of questioning, the control question test and the guilty knowledge test.  The control question test utilizes three different types of questions, normal questions, control questions and relevant questions.  The guilty knowledge test operates under the principle that subjects possessing guilty knowledge will have a strong physiological response when confronted.  Although we originally intended to test both of these methods of questioning, we chose to only use the guilty knowledge test in order to limit the time of questioning and keep the questioning as professional as possible.  The questioning environment is a laboratory test as opposed to a field study test. This means that the subject will be assigned the role of innocent or guilty and his job will be to convince the interrogator of his innocence and respond to questions as though an innocent or guilty subject would in the same circumstances.

There will be 12 subjects tested, six males and six females, assigned either a guilty or innocent role.  The test will consist of 28 questions, 5 for the stimulation task and 23 for the guilty knowledge test.  The experiment will take place in two isolated rooms.  The interrogator and polygraph reader will be in one room while the other lab members will be in the control room.  Before the experiment starts, the subject will be assigned to play the role of either an innocent or a guilty suspect by the lab members not affiliated with the questioning phase of the experiment.  The group members will also instruct the subject to give a negative response to all the guilty knowledge questions, and then they will set up the crime scene.  Based on whether the subject is assigned an innocent or guilty role he will “steal” an object of his choice from a designated area in the office.  He will then proceed to the interrogation room.  Here the interrogator’s task will be to decide whether the subject is innocent or guilty.  The interrogator will first talk to the subject and relax him. Before this occurs the subject will be attached to the polygraph.  He will be instructed not to make any auxiliary movements, the thermistor will be attached under his nostril, and his mouth will be taped shut so breathing can only occur through the nose.  The normal breathing rate will be recorded at this time.

The questioning will start with the stimulation test.  In the stimulation test the subject will pick a card from a deck and be asked to remember the card.  The subject will then be shown five cards in the desk and asked to identify his card.  The subject responds to each question with a card labeled yes and no that he will point to, in order to minimize body movement.  Each time a card is shown the subject is instructed to respond “no”.  At the end, the interrogator will properly identify the subject’s card based on the polygraph data because when the subject is lying the polygraph should show some physiological response.  In order to keep this portion of questioning constant, the subject was always shown his card on the fourth question.  The purpose of the stimulation task is to make the subject believe that the polygraph is accurate, only then can proper results be obtained.  More importantly, the stimulation task records the physiological “fingerprint” for each subject’s false response.

The questioning will continue with the 23 guilty knowledge questions, in five sets of four or five choices, regarding the crime committed.  Like the stimulation task, the subject is instructed to answer negatively to each question.  When the questioning is over the polygraph results will be examined.  The interrogator and polygraph examiner will score the polygraph tests using the numeric approach.  For each set of questions a score of 1 will be given for mild physiological differences when confronted with guilty knowledge, visible discomfort will give a score of 2 and obvious differences in physiological response will be given a score of 3.  If the subject receives a total score of 3 or higher for the sum of the questions he has failed the guilty knowledge test.  The interrogator will pool these results and then meet with the rest of the group to analyze the data further.  Each question will be examined and the percent error of the polygraph will be assessed based on the true knowledge of the other group members.  The overall percentage error of the test will be calculated among all of the 12 subjects when testing ends, as will the overall percentage error of the stimulation task and guilty knowledge test.

PRELIMINARY STUDIES:

The preliminary data that we will collect will give us three piece of information necessary to carry out our experiment.  With the preliminary data we will determine how the thermistor should be attached, we will create a calibration curve for the thermistor and determine the “a” and “b” values, and we will make sure our circuit works.

After three trials of exposing the thermistor to different water temperatures and recording the corresponding voltages from the multimeter, we used the temperature.vi to find the values of “a” and “b” in the Steinhart-Hart equation.  The “a” and “b” were determined to be a= 0.00391 and b=0.000244, with a standard deviation of 3.57x10-5 for “a”, and 1.45x10-5 for “b”.  These values will be plugged back into the equation in our experiment to obtain the corresponding temperatures from the voltages recorded by the polygraph virtual instrument for each subject.

We attached the thermistor in two different ways: to the skin under the nose and directly under the nostril, with no skin contact.  Based on two trials of two minute breathing, the more sensitive attachment was determined to be directly under the nostril, with no skin contact.  This is the method we will use next week for the actual experiment.  When the thermistor made contact with the skin, we were unable to obtain a breathing signal.

We decided to use the simple bridge in this experiment instead of the voltage comparator circuit from experiment 3 that we had originally planned.  This change was made during the course of our preliminary studies because the voltage comparator circuit was unnecessary in this experiment when coupled with the polygraph.vi.

In order to prove that our polygraph was functioning correctly, we conducted the stimulation task on one of our group members and showed that our circuit worked because her breathing rate was accurately recorded by the lie detector (see Figure 1).  This proves that our polygraph will be successful in determining false statements based on physiological responses.  The graph for the stimulation test shows a dip at question four (the fourth tick mark on the axis above the waveform) indicating a change in the shape of the characteristic waveform when she lied.  This change can take the form of either a change in the slope or a change in the time between breaths depending on the person.  In this case the change in the waveform is a slight drop in the amplitude of the wave.

Figure 1: MS

RESULTS:

It was found that the lie detector had an 87% accuracy rate when the stimulation task worked and a 58.3% accuracy overall.  The stimulation task worked 66.6% of the time.  The interrogator score was dependent on whether the stimulation test worked.  A change in the characteristic waveform for each subject indicates deceit.  The stimulation task was 66.6% effective in showing each subject's characteristic change for a lie.  After the characteristic change was recorded, the data was analyzed by matching up the characteristic waveform for deceit with the rest of the questioning.  The type of change in the characteristic waveform varied from person to person, and was sometimes characterized by solely a change in frequency (9% of the subjects), sometimes just change in magnitude (25% of the subjects), but most often (66% of subjects), the defining point for deceit was a change in the waveform's magnitude and frequency.

When 12 subjects were tested regarding their behavior in a fabricated crime and evaluated in a blind test the following results were obtained (see Figure 2).

Figure 2:

| Subject | Interrogator Score | Actual Score | Lie detector correct? | Stimulation Test Work? | Waveform change |
| --- | --- | --- | --- | --- | --- |
| 1-NT | -- | innocent | No | No | none |
| 2-AR | 4-guilty | guilty | yes | Yes | Freq/Mag |
| 3-AK | 4-guilty | guilty | yes | Yes | Freq |
| 4-SB | 1-innocent | guilty | No | Yes | Freq/Mag |
| 5-JK | -- | innocent | No | No | none |
| 6-SD | 5-guilty | guilty | yes | Yes | Freq/Mag |
| 7-JB | -- | innocent | No | No | none |
| 8-LL | 2-innocent | innocent | yes | Yes | Freq/Mag |
| 9-AM | -- | guilty | No | No | none |
| 10-KN | 1-innocent | innocent | yes | Yes | Mag |
| 11-AP | 5-guilty | guilty | yes | Yes | Freq/mag |
| 12-AX | 2-innocent | innocent | yes | Yes | Mag |

Upon analysis of respiration rate and breath temperature change as indicators for deceit the following graphs were obtained (see Figures 3 and 4).  The inversion of the breath temperature graph is due to the (1/Tk) result from the Steinhart-Hart equation.  Breath temperature values were obtained from the equation using the Vo value from the Lie Detector.vi and the a=0.00391 b=0.000244 of the Steinhart Hart equation during the preliminary studies.

Figure 3: Respiration                                         Figure 4: Temperature of Breath The graphs for both respiration and breath temperature produce the same waveform.

Waveforms were collected for all subjects using the Lie Detector.vi and show the respiration rate and breath temperature data from the thermistor.

Figure 5 represents a guilty subject waveform.  The abnormality found during the stimulation task (mark 4) carried through for each of the guilty knowledge questions.  This subject received a polygraph score of 5 and was indeed guilty.  The circles on the graph highlight the guilty waveform.

Figure5:APFigure 6 demonstrates an innocent subject waveform.  A characteristic false response was observed during the stimulation task (mark 4).  This response was not observed during the remainder of the readout from the thermistor.  One uncharacteristic abnormality was noted (the second circle), but this was not enough to alter the innocent verdict for the subject.  In reality, the subject did not remove anything from the room.

Figure 6: KN

The wave prints for the remaining subjects were evaluated in the same manner as the above subjects.  These examples (see figures 5 and 6) are the best results obtained.  Cases in which the stimulation task did not work produced a waveform such as the one below (see figure 7).

Figure 7: AM

The Stimulation Task did not work in this case.  There is no change in waveform during the false statement (mark 4).  Therefore, there is no established basis to determine false responses for the Guilty Knowledge Test.

DISCUSSION:

The major findings of our experiment were a) respiration rate and breath temperature were accurate indicators of deceit, b) selecting either respiration rate or breath temperature alone would also be a correct indicator of deceit because the graphs were identical, c) the stimulation task proved vital in data analysis because failure of the task would eliminate the possibility of analyzing the guilty knowledge test, and d) with a working stimulation task, which occurred 66.6% of the time, the polygraph had 87% accuracy in determining guilt or innocence.

Polygraph experiments in the past have produced varying degrees of accuracy.  Frank Horvath—a former president of the American Polygraph Association was quoted saying that “polygraph test accuracy is debatable”, but added, “most professionals believe the tests have between a 70 percent and 90 percent accuracy rate.”2  The Journal of Applied Psychology surveyed the topic in 1997.  At that time, the accuracy of polygraph testing was evaluated at only 61%.3  Based on these observations, our polygraph was within the range found by other experiments in the same area.

Our experiment was limited in several ways.  First of all, most expert polygraphs will test all five physiological indicators, whereas we only tested respiration rate and its resulting breath temperature.  We were also limited in using one test of the polygraph—the Guilty Knowledge Test.  However, of all the questioning options, this test is considered “a sound forensic tool,” according to the American Psychological Association.  This test does have its constraints however.  “One major problem with this test is that it has no controls. Also, unless the investigators have several pieces of insider information to use in their questioning, they run the risk of making a hasty conclusion based on just one or two "deviant" responses.”4  Perhaps the most significant limitation of our experiment was the fact that it was a laboratory test.  Because we were not questioning the subjects on an actual crime, the response data we obtained was dependent on how well the subject could convince himself of his role.

Initially we decided to use a comparator circuit in this experiment; however, this type of circuit was unnecessary.  After the circuit was built and connected to the computer, we realized that the LED’s were extraneous and the polygraph virtual instrument alone could give comprehensive data.  With the polygraph.vi the breathing rate could be determined properly based on the graphs and data collected by the software.  We chose to use a bridge circuit to replace the comparator circuit because the bridge gave clearer graphs (more noticeable dips and peaks) than the half bridge circuit did when tested. Also, since there was less noise that would affect our data by using the bridge circuit, we obtained more accurate graphs than the half bridge circuit.

Respiration rate and breath temperature were shown to be accurate indicators of deceit.  The waveform obtained by the polygraph virtual instrument was used to analyze the respiration rate data.  The individual voltage readings were also taken from the thermistor data, and were then plugged in to the Steinhart-Hart equation to obtain the temperatuare data.  When the respiration rate graph and the temperature data graph were compared it was found that they were the same.  The resulting graphs reflect a direct relationship between the two measurements.  The breath temperature graph above is the direct result of the Steinhart-Hart equation, which is 1/Tk.  This explains the inverse of the graph.  The direct relationship between respiration rate and breath temperature can be explained because the greater the respiration rate, the more the subject breathes out and the hotter the thermistor gets.  Our overall effectiveness of the polygraph was based on the two measurements, but the same results could be obtained independent of the method chosen.  Since our polygraph showed a high percentage of accuracy, 87%, based on the respiration rate/ breath temperature measurements, the physiological responses we chose to test were in fact accurate indicators of deceit.

In analyzing the data, problems arose if the stimulation task failed to produce an obvious physiological false response.  Since the purpose of the stimulation task was to create a waveform that would be used to identify later false responses, failure at this point prevented any further analysis for the subject.  We originally proposed that the stimulation task was 100% effective in creating a characteristic waveform, but disproved this thought on later analysis.  The reason for the change in thought was taking into account the conformation bias on the side of the interrogator.  The choice for the false response was held constant at the fourth card for all subjects.  While this limited the time it would take to determine the characteristic waveform, and confirm what the false response would be, we felt that in some cases the analysis in the lab selected a fourth card waveform identical to other wave forms included in true responses.  After second analysis, the stimulation task was determined to be 66.6% effective.  For the twelve subjects analyzed, the stimulation task failed four times, preventing further polygraph analysis for those subjects.

There were a few sources of error in this lab that could be eliminated in the future.  During the questioning the subjects were asked if they placed the stolen object in their purse.  Often, when a male was being questioned, this produced laughing and agitated respiration, which affected the results.  Laughing could be mitigated by being more sensitive to the questions asked and trying to screen for questions that could produce a humorous response.  It becomes even more important for the interrogator to remain stoic during questioning to try to keep the subject from laughing.  Another problem that arose during analysis of the data was that sometimes it was hard to distinguish the different waveforms because the questions were asked too fast.  In the future the interrogator should take extra care to ask questions slowly to enable the subject to have a complete breathing cycle during each question, which is necessary for analysis.

Analysis of the waveforms and determining guilt in the experiment was also problematic.  Determining the guilty response and the scoring of the guilty knowledge test was more qualitative than quantitative and often it was hard to determine whether the stimulation task worked.  Originally the stimulation task was thought to be 100% effective but after further analysis it was shown that the stimulation task was only 66.6% effective.  This was because the characteristic guilty response was hard to find with just qualitative parameters.  In the future it might be useful to have some quantitative measure to find the characteristic guilty response.  FFTs could be used to analyze the waveforms and power spectra could be generated to find the characteristic guilty waveform and quantitatively show whether the stimulation task worked or not.  The FFT would show that the characteristic guilty response occurred repeatedly at a given frequency and magnitude throughout the questioning.  This would be a more quantitative approach than looking at the graph and trying to see differences in the waveform.

Virj, Aldert.  Detecting Lies and Deceit.  John Wiley and Sons Ltd; West Sussex, England. 2000 (170-1).

2 Seper and Tradeway.  The Washington Times.  14 July 2001.  http://www.dailyhowler.com/h072501_1.shtml

3 Koerner, Brenden.  “Lie Detector Roulette.”  Mother Jones .  November/December 2002.  http://www.motherjones.com/news/feature/2002/43/ma_148_01.html

4 Carroll, Robert.  The Skeptics Dictionary.  http://skepdic.com/polygrap.html