# PROJECT  REPORT COVER PAGE

GROUP NUMBER____T4______

PROJECT  NUMBER___1P4___

TITLE_ Membrane Dialysis of Sodium Chloride and Sodium Polyacrylate _

DATE SUBMITTED__10/10__

# ROLE ASSIGNMENTS

ROLE                                                               GROUP MEMBER

FACILITATOR………………………..__Frank Lien____________

TIME & TASK KEEPER………………__Patrick Antkowiak_____

SCRIBE……………………………….._Adnan Aziz____________

PRESENTER………………………….___Jamil Beg______

# Summary

Over the last four weeks, the mass transfer coefficients for sodium chloride, sodium polyacrylate, and potassium chloride were determined experimentally using a time trial dialysis apparatus, which measures conductance on each side of a semi permeable membrane.  These conductances are converted to concentrations using a standard calibration curve of concentration versus conductance.  The Km values of KCl and poly-(sodium acrylate) are -.00222 + 7.34% cm/s (average value) and -0.00009928+ 6.93% cm/s, respectively. Using an osmotic pressure system an attempt was made to determine the dissociation constant for sodium polyacrylate; this system failed, so another method was implemented and a Ka of .00249 M was determined.  Of its 55 sodium ions, it is expected that 41.6 dissociate in solution.  The trials were optimized by using minimum volumes of solutions (just enough to fill the apparatus); the volume of solution is the largest factor in determining how long each trial will take.

Objectives.

To calculate the mass transfer coefficients, Km, of sodium chloride, sodium polyacrylate, and potassium chloride using data recorded from a dialysis system which records conductance of a solution using electrodes and a standard curve of known concentrations and experimentally determined conductances.

To experimentally determine the dissociation constant for sodium polyacrylate using an osmotic pressure system.

To optimize trial efficiency, reducing the time necessary to achieve results to a minimum.

Background

Dialysis is a mechanism by which much of the body’s necessary activities take place. Perhaps the clearest example of this is the kidney’s filtration activities. In basic dialysis essentially a semi-permeable membrane of a sort separates two different compartments of different concentrations and overtime molecules of lower molecular weight are allowed to pass through while those of higher molecular weight are blocked. Ions will move through the membrane in order to reach the side with lower chemical potential until both sides come to equilibrium and there is no longer a more favorable side.

Osmosis is a simple yet ingenious phenomenon, which has many possible mechanisms. In some cases, a semipermeable membrane may act as a molecular sieve, allowing smaller solvent molecules to pass through while blocking larger solute molecules. In other cases, osmosis may be caused by the higher solubility of the solvent in the membrane than the solute. Osmotic pressure measurement experiments are more sensitive ways to determine molar mass than are freezing or boiling point experiments.

Theory and Methods of Calculation

In order to find km and predict Ch, an equation was derived starting from a mass balance on the dialysis system:

ChVh+ClVl=m, derivate

Accept that Vh*dCh/Dt=mx,

plug into the equation from the manual, mx=km*A*(Ch-Cl)

Solve for Ch as a function of time and some constants

ln((2*Ch-Ct)/(2*Cho-Ct)) =2*Km*A*t/Vh

By plotting the left side as y, the right side as x, it was possible to solve for km and predict the concentrations as follows:

Ch=[(2*Cho-Ct)*e^(m*t)+Ct] / 2

Where m is the slope of the above plot and equals 2*km*A/Vh

In the osmotic pressure portion of the dialysis experiment we used an equation relating the osmotic pressure and static pressure.

- pressure = force/area = hg = vMRT

v is for the amount of ions disassociated

The formula relating pressure and osmotic pressure and an example problem on pg 238 in Physical Chemistry by Chang laid out the quantitative explanation behind the osmotic pressure experiment performed in this lab.

Materials, Apparatus, and Methods

4-Plastic pipets with ends sawed off cleanly and pipet stands

2 beakers

Dialysis tubing

Oversized syringes

And materials listed in the BE Lab  manual

Methods beyond those listed in the BE Lab Manual:

When setting up the dialysis system using the Brookfield viscometer, syringes were used to extract air bubbles, which often caused problems for the sensors, creating massive and quick rises and drops in the data.

When conducting the part of the experiment relating osmotic pressure to concentration, it was decided to use a system of burets partially immersed in a beaker of water, with dialysis tubing tightly and securely fastened between the two compartments. The height of the liquid in the buret was checked after an hour, a day and a week.

Results

Figure 1 Depicts the calibration of the KCl probe used in weeks 1 and 2.  Such calibrations were performed in all weeks regardless of the compound used in the dialysis; this is just a representative.  Note that at high concentrations (~.6M +) the graph starts to deviate from its linearity.  In the experiments conducted, the maximum concentration that was used was .1M, so it was possible to assume the concentration calibration curve was linear over the portion used.  Additionally, the maximum difference between probes over this region was 1.15%, so the effect of using different probes during was minimal compared with other uncertainties encountered in dilutions and other measurements.

Figure 2 depicts the method by which the mass transfer coefficient Km was determined.  By plotting ln((2*Ch-Ct)/(2*Cho-Ct)) vs. 2*Km*A*t/Vh, it is possible to solve for Km, using the slope determined from the graph some measurable quantities.  A is the area available for transport in cm2, Vh is the volume of solution in mL on the higher side of the dialysis concentration gradient, Ct is the sum of the concentrations of the species in the dialysis, Ch is the concentration of the species on the higher side of the dialysis concentration gradient, and Cho is Ch evaluated at t=0 minutes.  The graph yielded a highly linear slope in all cases.

Figures 3-6 depict the dialysis experiments run in weeks 1-4, respectively.  In the first three weeks of the experiment, it was possible to model the behavior of the dialysis system by solving a mass balance differential equation; this is included in the graph along with the measured concentrations for each week’s experiment.  Qualitatively, the first two graphs resemble each other.  Figure 4 is a vertical shift up and a vertical compression of Figure 3.  The only parameter varied from week 1 to week 2 was the concentration on one side of the membrane: flow rate was constant at about 7.6 mL/min, volume per side was around 13.5 mL (exact volumes in table 1) and flow orientation remained constant week to week.

Figure 5 represents a dialysis performed with poly-(sodium acrylate) on one side and distilled water on the other.  It is apparent from the graph that the concentration on either side of the membrane changed very little.  The Km value of this dialysis was significantly smaller than in the first two weeks (see table 1).

The models created as described above were serviceable predictors of the concentration on either side.  In weeks 1-2, the models differed from the measured quantities by about 10%.  However, the model in week 3 was within 2% of the actual concentration, and was a much better predictor of concentration.

Figure 6 shows the dialysis system consisting of  .1M NaCl on one side of the membrane and .1% w/w poly-(sodium acrylate) on the other side.  The graph seems qualitatively similar to the KCl dialyses on the top half and similar to poly-(sodium acrylate) on the bottom half.

Table 1 includes all the data used in the dialysis experiments to set up the cell model, and the Km values calculated from the data.  The km values from week 1 and week 2 differ by only 3.67%, which is less than either of their uncertainties; thus, they are not significantly different from each other.  The majority of the uncertainty from determining the km value arose due to the lack of precision in measuring the area available for transport in the dialysis system.  It was not possible to create a useful model of the dialysis system using known methods in the last week of the experiment.

Table 2 is a comparison of the percent difference in Km if it were calculated using the time T on the right column instead of using the total experiment time (in this case, 93 minutes).  Using only 10 minutes of time still allows for a pretty reasonable value of Km.  However, since the uncertainties in Km were around 7%,  it is recommended that data points be taken for at least 30 minutes to determine the Km of the experiment, since the difference between Km at 30 minutes and Km during 93 minutes is about 7%.  This corresponds to about 1/3 of the total time planned for the experiment.

Since the apparatus for the osmotic pressure apparatus failed, another method was needed to determine the Km of poly-(sodium acrylate).  Conductance is roughly proportional to the number of ions in solution, so comparison of the calibration slopes of  NaCl and poly-(sodium acrylate) made it possible to determine about how many ions dissociate for poly (sodium acrylate).  The conductance slopes were 7.28 mV/ C and 151.55 mV/ C, respectively, for NaCl and poly-(sodium acrylate);  dividing the results yields a ratio of  1 : 20.8, which implicates that 20.8 times more ions dissociate from poly-(sodium acrylate) than NaCl.  Since NaCl strongly dissociates into 2 ions, then poly-(sodium acrylate) dissociates, on average, into 41.6 ions.  Applying the equation:

Ka=   c*a^2 / (1-a)

Where c represents the concentration and a represents the percent dissociation of ions, it is possible to obtain a value for Ka.  A mer of poly-(sodium acrylate) consists of 55 ions, so Ka at the concentration of .00106 M used in week three is  .00249 M.

Discussion

The mass transfer coefficients (km) are -0.00009928 for sodium polyacrylate and -0.002218 for potassium chloride.  The km for sodium polyacrylate is a deviation of only -0.6541% in the concentration gradient from a theoretical value found using a model derived from the rate of diffusion across a dialysis membrane equation.  The km value for potassium chloride deviated on an average of -10.66%.   Considering the precision of our km values for potassium chloride, which only differ from each other by approximately 3.75%, the deviation can be disregarded and accredited to variability in instrumentation. In determining the rate of flow of ions across the membrane, three variables are taken into consideration: the mass transfer coefficient, area of the membrane exposed to diffusing solution, and the concentration gradient across the membrane.

Part of the experiment involved investigating optimizing the procedure in which to run dialysis trials.  Increasing the surface area of diffusion (of the membrane) would increase the rate of diffusion, but is not feasible with the current apparatus.  Minimizing the time of each trial would allow for more trials performed per day, this is accomplished by decreasing the concentration gradient.  This essentially positions the start further up on the conductance vs. time graph.  The most effective way to minimize the time is to limit the volumes of the two solutions by closing off the system.  In this case, since the least volume is used, the moles of ions available to diffuse is minimized, capping off the diffusion time to its shortest length.

These km values though tell nothing of the amount of dissociation that actually occurs due to the concentration gradient.  A comparison of the standard curves for sodium polyacrylate and sodium chloride reveals that sodium polyacrylate dissociates approximately 40 times more ions than the sodium chloride which completely dissociates in solution.  This does not explain the relatively low km value that sodium polyacrylate provides in contrast with sodium chloride’s high km value.  The slow diffusion of sodium in sodium polyacrylate across the membrane may be due to the relatively large size of polyacrylate.  The polyacrylate can not diffuse through the semi permeable membrane; instead, it’s great size blocks off the passages in the membrane which the sodium would use to pass through to the side with less concentration.  With the area of diffusion greatly minimized, diffusion slows down tremendously.  Another factor contributing to the lack of diffusion across membrane is electrochemical charge. Since it is necessary to maintain electrochemical neutrality in solution, and the polyacrylate consists of a negative charge, while sodium is positively charged, the polyacrylate exerts a stronger electronegative pull than the osmotic pull of the concentration gradient.  Therefore, allowing only a little sodium to actually pass through the membrane into the lower concentration solution.  The molecular weight of sodium polyacrylate is 5100 g/mol, with one unit being but 94 g, there is approximately 55 sodium ions per mole of polyacrylate capable of dissociating.  From what we found, 41.6 of these should be capable of diffusing across the membrane.

To test this hypothesis, an osmotic pressure experiment was set up.  Assuming that approximately 40 sodium ions would diffuse across the membrane per mole of sodium polyacrylate, four concentrations were used.  Dilutions of 1.0% w/w and 0.05% w/w were expected to overflow the 27 cm height allotted for the experiment.  A 0.0125% w/w dilution was expected to just reach the 27 cm height while a 0.00625% w/w dilution was expected to reach a 13.5 cm height.  After a week, the solutions showed no significant increase in height.  The reason for this is that the surface area of diffusion was too low.  A similar experiment performed by the lab technician showed that by increasing the surface area of diffusion, the speed of diffusion increases dramatically, although still not sufficiently enough for it to reach equilibrium within a week.  Also it may have been possible that there was a concentration gradient within the solution so that the sodium polyacrylate closest to the membrane did not have any more diffusible ions, therefore significantly slowing down diffusion by further reducing the surface area of diffusion (lowering the probability that sodium ions would be in contact with the membrane).  Mixing the solutions periodically would have resolved this issue.  In addition, there may have been a leak in the membrane or where the membrane was connected to the pipette.  This would be evidenced by an abnormally high conductance reading in the water, which was not tested due to lack of time.

The osmotic pressure procedure of the experiment may experience greater success if the solution placement is inverted, meaning the height measured would be of the water.  To ensure a more stable flow of ions the beaker could be placed on a magnetic stirrer.  Instead of diluting the sodium polyacrylate to a great extent, which inevitably results in human errors, a longer rigid tube could be used to measure height of solution.  Since it also takes so long for the osmotic pressure experiment to exhibit significant data, it may be better to start that part of the experiment at an earlier week.  A larger dialysis apparatus would help in cutting down the typically long dialysis run time.  Also, a separate reservoir could be created in the dialysis system to collect the air bubbles that inevitably form in the creation of a closed system.  These air bubbles can greatly influence the conductance readings if they get caught between the plates in the conductance meter and can reduce the surface area of diffusion if they gather along the dialysis membrane.

Conclusions

1. The mass transfer coefficients, Km, of potassium chloride and sodium polyacrylate are -0.002218 cm/s and -0.00009928 cm/s, respectively.   Models made using these km values result in concentrations that differ from ones measured in the lab under the same conditions by  -10.66% and             -.6541% respectively.

2.  Sodium polyacrylate dissociates 41.6 of its 55 Na+ ions in solution with a dissociation constant of  .00249 M.

3. The trials are optimized by using a minimum volume of solution in a closed system.  Concentration differences were observed to only minimally affect the time it took to run the dialysis experiment.

References

Chang, Raymond. Physical Chemistry. Sausalito, CA. University Science Books, 2000

http://www.pitt.edu/~patzer/dialysis/modelingdialysis.htm

Figure 1: KCl probe calibration

Figure 2:  Determination of Km

Figure 3:  KCl/H2O Dialysis (Week 1)

Figure 4: KCl/ H2O Dialysis (Week 2)

Figure 5:  Poly-(Sodium Acrylate)/ H2O Dialysis

Figure 6:  NaCl/ Poly-(Sodium Acrylate) Dialysis (Week 4)

Table 1:  Data and Uncertainties of Dialysis Experiments

|  | Area(cm^2) | Vh (mL) | Ct (M) | Cho (M) | km (cm/s) |
| --- | --- | --- | --- | --- | --- |
| Week 1 | 18.3 | 13.425 | 0.104 | 0.104 | -0.00218 |
| Uncertainty | 6.70% | 0.07% | 1.34% | 1.34% | 6.96% |
| Week 2 | 18.3 | 13.512 | 0.152 | 0.1 | -0.00226 |
| Uncertainty | 6.70% | 0.07% | 2.68% | 1.33% | 7.34% |
| Week 3 | 18.3 | 13.456 | 0.00106 | 0.00106 | -0.0000509 |
| Uncertainty | 6.70% | 0.07% | 1.75% | 1.75% | 6.93% |
| Week 4 | 18.3 | 14.601 | 0.116 | 0.115 | ----------- |
| Uncertainty | 6.70% | 0.07% | 2.90% | 1.15 | ----------- |

Table 2:  Percent difference of Km at time T vs. Km at time T=93 mins (total time of experiment)

| % Difference | Time (mins) |
| --- | --- |
| 12.00% | 10 |
| 10.34% | 20 |
| 7.81% | 30 |
| 5.42% | 47 (half of total time) |
| 0 | 93 (total time) |

http://www.pitt.edu/~patzer/dialysis/modelingdialysis.htm

Chang (pg 238-240)