# LABORATORY REPORT COVER PAGE

GROUP NUMBER & NAMES W1: Michelle Gupta, Jennifer Fleischer, Jonathan Abelson, Andrew Franklin, Hyun Tae Shin

PROJECT NUMBER 3

TITLE Cyclic Testing of Wooden Material

DATE PERFORMED March/April 2004

AUTHORIZED SIGNATURE ___________________________________

Brief Background:

Material and structural properties of different materials can be evaluated by performing cyclical loading tests. The cycle life of a material helps to determine its performance characteristics. This is essential when deciding what materials to use in products that are subject to constant cyclic loading such as knee braces, artificial tendons, and replacement heart valves.  It is important to recognize that within cyclical loading there is a difference between low and high cycle fatigue.  Low cycle fatigue is cyclic loading with a maximum load that is below the static ultimate structural failure load, this includes maximum loads that are just below the ultimate failure load.  High cycle fatigue is defined as cyclic loading with a maximum load that is much below the static ultimate structural failure load; this type of loading takes a long amount of time since the material experiences relatively small loads.  Due to time constraints only low cycle fatigue was investigated. A three point bending system was used to observe the effects of cyclic loading on materials in this project.

Hypothesis/Objectives/Aims:

The principle objective of this project was to evaluate and determine fatigue strength of wooden materials.  To accomplish this it was decided that a relationship between support length and cycles to failure be determined along with a relationship between maximum load and cycles.

The hypotheses of this project were that the fatigue strength of the wooden sample would increase as the distance between the two bottoms supports increased, and that as the load increased the number of cycles to failure would decrease.

General Protocol:

Fatigue strength was evaluated through cyclic loading of the wooden tongue depressors using a three point bending system.  The effects on the fatigue strength of the wooden samples were evaluated by cyclically loading the samples with different variables.  First, the distance between the two bottom supports was altered.  The wood’s fatigue strength was tested using three different support lengths.  Fatigue failure was defined as a 10% increase in extension, plastic deformation.  Normal (taken directly as is from the box) wooden samples were used in this experiment.  Secondly, a pseudo SN curve was generated.  A constant support length was used on a normal wooden sample in cyclic loading.  The max load on the sample was varied and samples were allowed to run until failure in order to determine the fatigue strength at each interval.

Specific Methods:

Selecting Wooden Samples: One person in the group selected each of the wooden samples for testing. While all inconsistencies in wooden samples could not be avoided, those wooden samples with large and visible kinks, grains, bending, uneven edges and other deformities were not used.

Due to time constraints on the experiment a 5min cap was set on all samples in the varying length experiment. Failure was designated as ≥10% creep. All samples tested came directly from the box: no water was added or taken away.

## Varying Length

- Adjust bottom supports to the short length (5cm)

- Set loading rate at 200mm/min

- Set data acquisition rate to 20 points/sec

- Set max load to 55N

- Run sample until failure at the set length or until time limit is reached (5 min)

- Repeat for 9 additional samples

- Adjust bottom supports to the medium length (7cm)

- Run sample until failure at this set length or until time limit is reached (5 min)

- Repeat for 9 additional samples

- Adjust bottom supports to the long length (9cm)

- Run sample until failure at this set length or until time limit is reached (5 min)

- Repeat for 9 additional samples

- Use chi squared test to compare failure rates of three sample sets (see appendix for details)

### Development of Pseudo S-N Curve

- Load samples at medium length (7cm)

- Set loading rate to 300 mm/min

- Set data acquisition rate to 30 points/sec

- Set max load rate, (starting at 40 Newtons)

- Load sample and run until failure (no time cap)

- Repeat above procedure for two additional samples

- Increase max load rate by 5 Newtons

- Repeat above steps until desired data has been obtained

- Generate curve, plot number of cycles to failure vs. force.

For the Pseudo S-N Curve the number of cycles to failure was obtained using the loading rate, the data acquisition rate, the average cycle distance, and the total number of points acquired.  With this information cycles to failure was calculated:

- Divide total number of points by the data acquisition rate = total time

- Divide loading rate by the average cycle distance = cycles / time

- Multiply cycles per time by total time to get the number of cycles

Results:

Varying support length

|  | Did Not Fail | Failed | Total |
| --- | --- | --- | --- |
| Short Observed | 7 | 3 | 10 |
| Medium Observed | 3 | 7 | 10 |
| Long Observed | 5 | 5 | 10 |
| Total | 15 | 15 | 30 |

Table 1. A 3x2 chi square table of our raw frequencies and totals

|  | Did Not Fail | Failed |
| --- | --- | --- |
| Short expected | 5 | 5 |
| Medium expected | 5 | 5 |
| Long expected | 5 | 5 |
| Table Value | 3.2 |  |
| Critical Value | 5.99 |  |

Table 2. Calculated expected frequencies and the table value versus the critical value

The Chi Square table (Table 1) was used to draw a conclusion from the data found in the different support lengths.  There were three groups of data: Short, Medium, and Long. The goal was to determine whether one group was statistically (p<0.05) more likely to fail under an applied maximum load of 55 Newtons with a crosshead speed of 200 mm/min.

The values in Table 1 represent the raw frequencies that were determined from testing, while the values in Table 2 represent the calculated expected values for how many times a given sample should or should not have failed.  The conclusion from the Chi Square test was that one group was not more likely to fail than any of the others.  Ultimately, no correlation was found between the support length and the fatigue strength of the wooden samples.

|  | Long | Medium | Short |
| --- | --- | --- | --- |
| Average | 7.13 | 6.73 | 6.70 |
| STD | 2.01 | 0.59 | 1.85 |

Table 3. Calculated average % deformation for samples that did not fail

The values in Table 3 represent the average percent creep achieved at each support length after 5 minutes of cyclical loading.  These were all samples that did not fail and experienced the same amount of testing.  All averages are within 1 percent of each other.  With their accompanying standard deviations the values are statistically not different (p>0.05).  This suggests that the amount of creep experienced by the samples that did not fail was independent of the support length.

Pseudo S-N Curve

Graph 1. Relationship between maximum load and number of cycles until failure

|  | Average Cycles | STD |
| --- | --- | --- |
| 40 N | 359.59 | 258.33 |
| 45 N | 100.16 | 81.34 |
| 50 N | 5.77 | 3.93 |
| 55 N | 3.77 | 1.20 |
| 60 N | 5.54 | 6.01 |
| 65 N | 2.62 | 0.66 |
| 70 N | 1.36 | 0.74 |

Table 4. Chart showing the average cycles to failure for the related maximum load.

The chart above shows that once the maximum force in the load cycle exceeds 45 N, the average number of cycles until failure drops below 10.  The minimum load was set constant for all trials at 25 N.  This trend is clearly illustrated in Graph 1, and leads to a distinction between the two groups.  All trials conducted exhibit an inverse relationship between force and cycles to failure. However, a linear regression of the points ≥ 50 N gives a slope of -.1997 with a correlation of .6984, while a linear regression of the first three data points is -35.382 with a correlation of .9324. This shows a weaker correlation between the points ≥ 50 N. It can also be seen that the standard deviation associated with the average number of cycles until failure radically drops when the maximum load is increased from 45 N to 50 N.  In the 40 N tests, one sample failed after 900 cycles while another failed after only 30, thus the large standard deviation of approximately 260.

Discussion:

In order to simultaneously analyze all of our data, a method of analysis was required that could handle two different types of data, those that failed and those samples that did not fail.  Because of this requirement normal T-tests or an ANOVA could not be performed on the entire data set.  Additionally, statistical testing between specific groups, ie. short that did not fail and long that did not fail, would be based on relatively small sample populations and might not be representative of the entire population.  Because of these considerations a chi-squared analysis was relied upon to determine the correlation between support length and fatigue life.  In this comparison the table value was compared to the critical value in Table 2 of the results. Since the table value was less than the critical value at the 95% confidence level, there was no difference between the three groups in the table. More specifically, there was no correlation between support length and frequency of failure for the study.

One thing that must be considered is the inherent variability in every wooden sample, regardless of the method used to pick “standard” samples.  Also, it is possible that if a larger sample size could have been achieved for each support length a statistical difference may have been seen. A qualitative trend was seen in the fatigue strength of the support lengths.  In the long support length, 2 broke on the first cycle and 1 broke on the second cycle.  The medium support length had a wide range of cycles to failure.  The short support length had a relatively large number of cycles to failure.

An average percent creep for the samples that did not fail was taken at each support length.  This was a good comparison, because the amount of time for cyclical loading was constant (5 minutes).  The average percent creep for each support length ranged from 6.7% to 7.1% (Table 3).  This supports the result that no correlation exists between support length and fatigue strength, because the average percent creep experienced for each support length were relatively similar.  It can be concluded that no correlation exists between the support length and the fatigue strength given the conditions used in this experiment.

This experiment also resulted in the generation of a ‘pseudo SN curve’.  A standard SN curve in which stress is plotted against number of cycles until failure could not be created since the available equipment presented no control for number of cycles.  Thus the relationship had to be modified; in Graph 1 cycles to failure was plotted against the independent variable force.  The instron machine allowed for cyclical loading in which the maximum and minimum load of the cycle could both be controlled.

There are two interesting aspects to the pseudo SN curve generated in this experiment.  The first is the general trend of the data, as was expected cycles to failure generally decreased as the maximum load increased.  This finding supported the initial hypothesis that cycles to failure and maximum load were inversely related.  However the rate of this decrease changes rapidly at the 50 N point.  The delineation that exists at this point creates two distinct phases of the graph.  The first encompasses the 40 N - 45 N and 45 N – 50 N transitions, the slopes of the lines between these points exhibit a rapid change in cycles to failure as load is increased.  Beyond the 50 N point this trend ceases, the change in cycles to failure as the maximum load is incremented by 5 N becomes relatively small, less than a 3-cycle decrease.  This suggests that there exists a threshold around the 50 N point, beyond this point changing the maximum load has a relatively small influence on the number of cycles to failure.  At 50 N it took an average of only 6 cycles to attain failure; with such few cycles to failure there is little room for decrease.

This threshold also represents a point of transition in the variability associated with the number of cycles to failure.  Trials done at 40 N and 45 N resulted in large standard deviations 260 and 82 respectively, suggesting a great degree of variability within the samples which is an expected result given the organic nature of the material.  The tongue depressors tested also demonstrated subtle variations in their cross sectional area (30.28mm2 + 1.78), thus with composition and size both variable, the inconsistencies in the sample’s fatigue strengths were understandable.  It also seems logical that as the maximum load is increased the variability becomes less apparent.  The greater force makes the small variations within the wood less important, and the general properties of the wood dominate its behavior.  This trend was demonstrated experimentally by the drastic decrease in standard deviations between the 45 N and 50 N trials.

Due to time constraints, water content was not evaluated in this experiment, although it was considered. A suggestion for future experiments would be to repeat this experiment, with the addition of sample sets of varying water content. Another suggestion would be to increase the size of sample sets which would allow for other more rigorous statistical testing to be performed.

Appendix:

References:

- Website providing information on chi squared analysis:

http://www.georgetown.edu/faculty/ballc/webtools/web_chi_tut.html

Brief overview on how to perform a chi squared analysis:

The following chart, data from the experiment, will serve as the example.

|  | Did Not Fail | Failed | Total |
| --- | --- | --- | --- |
| Short Observed | 7 | 3 | 10 |
| Medium Observed | 3 | 7 | 10 |
| Long Observed | 5 | 5 | 10 |
| Total | 15 | 15 | 30 |

- First expected values need to be calculated for both conditions (Did not fail / Failed) of the three data groups, short, medium and long.  To do these calculations multiply the column total by the row total and divide by the matrix total.

For short did not fail: (15*10) / 30 = 5

- After calculating the expected value (Coincidentally for this data all expected values turn out to be 5) for all combinations a new graph is constructed representing the expected outcomes.

|  | Did Not Fail | Failed |
| --- | --- | --- |
| Short expected | 5 | 5 |
| Medium expected | 5 | 5 |
| Long expected | 5 | 5 |

- Using the expected and the observed data a table value, similar to a t-calc in a T-test can be calculated for the original observed data table. The table value is determined by the formula:

Σ (O-E)2

E

O is the observed value and E is the expected value of the cells from the corresponding tables.

- Finally the critical value is referenced from a table using a confidence level and a degree of freedom calculated by: (r-1)x(c-1), where r is the number of rows and c is the number of columns in table 1.  If the table value is greater than the critical value the null hypothesis, that the values are the same, is rejected and it can be concluded that the data sets are statistically different.  In this example the table value of 3.2 is less than the critical value of 9.49, thus the data sets are statistically not different.